##### A vehicle to vehicle communication protocol for co-operative collision warning.

ABSTRACT

Our Project proposes a vehicle-to-vehicle communication protocol for cooperative collision warning. Emerging wireless technologies for vehicle-to-vehicle (V2V) communications are promising to dramatically reduce the number of fatal roadway accidents by providing early warnings. One major technical challenge addressed in this our project is to achieve low-latency in delivering emergency warnings in various road situations. Based on a careful analysis of application requirements, we design an effective protocol, comprising congestion control policies, service differentiation mechanisms and methods for emergency warning dissemination. Simulation results demonstrate that the proposed protocol achieves low latency in delivering emergency warnings and efficient bandwidth usage in stressful road scenarios. The wireless data communication between two vehicles is provided by introducing Zigbee technology. It is designed around low-power consumption allowing batteries to essentially last forever. The distance measurement is provided by Ultrasonic sensors. Ultra sonic sensors are transmitting and receiving ultrasonic signals

TABLE OF CONTENTS

CHAPTER  NO.		TITLE 		 		                  PAGE

LIST OF TABLES

LIST OF FIGURES

LIST OF ABBREVIATIONS

1			INTRODUCTION					1

2			BLOCK DIAGRAM					2

3			ULTRASONIC SENSORS				3

3.1 FUNCTION

3.2 DISTANCE MEASUREMENT

4			MICROCONTROLLER 89C51				                                                 4.1 FEATURES

4.2 DESCRIPTION

4.3 ARCHITECTURE

4.4 PIN DIAGRAM

4.5 PIN DESCRIPTION

4.6 ABSOLUTE MAXIMUM RATING

5			  ZIGBEE

5.1 INTRODUCTION

5.2 WORKING PRINCIPLE

5.3 TYPES OF ZIGBEE

5.4 AREAS OF APPLICATION

6			 BUZZER

6.1 INTRODUCTION				6.2 WORKING PRINCIPLE

7			 SCHEMATIC DIAGRAMS

7.1 ULTRASONIC SENSOR

7.2 ZIGBEE

7.3 MICROCONTROLLER

7.4 MAX3232EPE

7.5 LD1117S33 SOT-233

7.6 TIP122

8			 PROGRAMMING 89C51

9			 DATA SHEETS

9.1 ULTRASONIC SENSOR

9.2 MICROCONTROLLER

9.3 ZIGBEE

9.4 MAX3232EPE

9.5 LD1117S33 SOT-233

9.6 TIP122

10			  REFERENCE

11			  CONCLUSION

BIBLOGRAPHY

CHAPTER 1

INTRODUcTION

Road accidents account for a severe threat to human lives from both an injury as well as a financial perspective. Given that vehicles are designed to facilitate a smooth means of transportation, manufacturers have long been in the process of designing vehicles based on principles of reliability and safety. However, due to reasons such as human-error, circumstantial error and negligence, accidents occur. Today, special attention is focused on the technologies that can reduce traffic accidents. V2V technologies are simple to implement primarily because of their reliance on wireless communication.

The communication protocol includes Zigbee to communicate the information between two vehicles. The distance measurement between two vehicles is done by Ultrasonic sensor. The microcontroller controls entire process, it is programmed to send a signal to buzzer and zigbee when the distance range is obtained. The main objective of our project is to alert the driver when he closes to the front vehicle. During night times some of the vehicles such as car, bus may break down at the highways. This vehicle now appears to be an obstacle to the vehicle that is coming behind of it. This causes a greater chances of accident, the vehicle coming behind may hit hardly to the back of stationary vehicle and it may lead to the greater damage.

CHAPTER 2

BLOCK DIAGRAM

CHAPTER 3

ULTRASONIC SENSOR

3.1 FUNCTION:

They are also known as transducers, they both send and receive the signal. It works on the principle similar to radar or Sonar which evaluate attributes of a target by interpreting the echoes from radio or sound waves respectively. They generate high frequency sound waves and evaluate the echo which is received back by the sensor. Sensors calculate the time interval between sending the signal and receiving the echo to determine the distance to an object.

Whenever a sound wave moving in air hits a solid surface it reflects . We often call  this reflected sound as an echo. The same applies to a sound wave moving through water and hitting an obstacle. If we know the speed of sound in the air or water we can calculate the distance. To perform this we must measure the time taken for a pulse of sound to travel to the object and back again. The distance to the object and back is given by (distance=Speed * Time). As this is the total distance that the sound has traveled to the object and back, we must divide by 2 to find the one-way distance. This use of echoes is the basis of SONAR. The pulse of sound that is used should be short and high frequencies are usually used as they travel further without being absorbed.  Sounds with a frequency above 20 kHz are called ultrasonic(beyond the range of human hearing). The speed of sound varies from one material to another. Typical speeds are approximately 330 m/s in air, 1500 m/s in water, 5000 m/s in a metal.  Here the ultrasonic sensor calculates the distance between the sensor and the obstacle and transmits it to the system. Once an object is detected, a signal is sent to the onboard sense and avoids collision and corrective action is taken by the use of micro-controller.

DISTANCE MEASUREMENT:

Different techniques can be used to measure the distance by using ultrasonic sensors. Among them , continuous-wave and pulse-echo technique are widely known. In continuous-wave methods, the transmitter generates a continuous output, whose echo is detected by a separate receiver. In this case, accuracy depends on the measurement of the phase shift between the transmitted and the reflected wave. Although better performance than with pulse-echo measurements can be obtained, complex hardware is required to measure the phase, and in most cases, different frequencies need to be used to determine the number of integer wavelengths in the phase shift.

Pulse-echo techniques are widely used in commercial systems due to less complexity of hardware. In pulse-echo technique, a short train of pulses is generated, enabling the same transducer to be used both as a transmitter and as a receiver. In the measurement methods, based on pulse-echo, the distance information is retrieved from a time-offlight measurement, i.e., the time an ultrasonic wave needs to travel from the transmitter to the receiver after being reflected by an object. The pulse-echo technique is used in this paper to calculate the distance of the obstacle from the vehicle. The distance between the transmitter and the object is determined using the following equation:

D= (t * c) / 2

Where D- Distance between the source and the obstacle

c- Ultrasonic wave velocity in air

t- Time interval between pulse emission and echo detection.

CHAPTER 4

Microcontroller AT89C51

4.1 FEATURES:

- Compatible with MCS-51™ Products

- 4K Bytes of In-System Reprogrammable Flash Memory– Endurance: 1,000 Write/Erase Cycles

- Fully Static Operation: 0 Hz to 24 MHz.

- Three-level Program Memory Lock

- 128 x 8-bit Internal RAM

- 32 Programmable I/O Lines

- Two 16-bit Timer/Counters

- Six Interrupt Sources

- Programmable Serial Channel

- Low-power Idle and Power-down Modes

4.2 DESCRIPTION:

The AT89C51 is a low-power, high-performance CMOS 8-bit microcomputer with 4K bytes of Flash programmable and erasable read only memory (PEROM). The device is manufactured using Atmel’s high-density nonvolatile memory technology and is compatible with the industry-standard MCS-51 instruction set and pinout. The on-chip Flash allows the program memory to be reprogrammed in-system or by a conventional nonvolatile memory programmer.

4.3 ARCHITECTURE:

4.4 PIN DIAGRAM:

4.5 PIN DESCRIPTIONS:

VCC-Supply voltage.

GND-Ground.

Port 0-It is an 8-bit open-drain bi-directional I/O port. As an output port, each pin can sink eight TTL inputs. When 1s are written to port 0 pins, the pins can be used as high impedance inputs it may also be configured to be the multiplexed low order address/data bus during accesses to external program and data memory. In this mode P0 has internal pull-ups. Port 0 also receives the code bytes during Flash programming, and outputs the code bytes during program verification. External pull-ups are required during program Verification.

Port 1-It is an 8-bit bi-directional I/O port with internal pull-ups.The Port 1 output buffers can sink/source four TTL inputs. When 1s are written to Port 1 pins they are pulled high by the internal pull-ups and can be used as inputs. As inputs, Port 1 pins that are externally being pulled low will source current (IIL) because of the internal pull-ups. Port 1 also receives the low-order address bytes during Flash programming and verification.

Port 2-It is an 8-bit bi-directional I/O port with internal pull-ups. The Port 2 output buffers can sink/source four TTL inputs. When 1s are written to Port 2 pins they are pulled high by the internal pull-ups and can be used as inputs. As inputs, Port 2 pins that are externally being pulled low will source current (IIL) because of the internal pull-ups. Port 2 emits the high-order address byte during fetches from external program memory and during accesses to external data memory that uses 16-bit addresses (MOVX @DPTR). In this application, it uses strong internal pull-ups when emitting 1s. During accesses to external data memory that uses 8-bit addresses (MOVX @ RI), Port 2 emits the contents of the P2 Special Function Register. Port 2 also receives the high-order address bits and some control signals during Flash programming and verification.

Port 3-It is an 8-bit bi-directional I/O port with internal pull-ups. The Port 3 output buffers can sink/source four TTL inputs. When 1s are written to Port 3 pins they are pulled high by the internal pull-ups and can be used as inputs. As inputs, Port 3 pins that are externally being pulled low will source current (IIL) because of the pull-ups.

Port 3 also serves the functions of various special features of the AT89C51 as listed below:

Port 3 also receives some control signals for Flash programming and verification.

RST-Reset input. A high on this pin for two machine cycles while the oscillator is running resets the device.

ALE/PROG-Address Latch Enable output pulse for latching the low byte of the address during accesses to external memory. This pin is also the program pulse input (PROG) during Flash programming. In normal operation ALE is emitted at a constant rate of 1/6 the oscillator frequency, and may be used for external timing or clocking purposes.  If desired, ALE operation can be disabled by setting bit 0 of SFR location 8EH. With the bit set, ALE is active only during a MOVX or MOVC instruction. Otherwise, the pin is weakly pulled high. Setting the ALE-disable bit has no effect if the microcontroller is in external execution mode.

PSEN-Program Store Enable is the read strobe to external program memory. When the AT89C51 is executing code from external program memory, PSEN is activated twice each machine cycle, except that two PSEN activations are skipped during each access to external data memory.

EA/VPP- External Access Enable. EA must be strapped to GND in order to enable the device to fetch code from external program memory locations starting at 0000H up to FFFFH. This pin also receives the 12-volt programming enable voltage (VPP) during Flash programming, for parts that require 12-volt VPP.

XTAL1- Input to the inverting oscillator amplifier and input to the internal clock operating circuit.

XTAL2- Output from the inverting oscillator amplifier.

4.6 Absolute Maximum Ratings:

| Operating Temperature | -55oC to +125oC |
| --- | --- |
| Storage Temperature | -65C to +150C |
| Voltage on any Pin with Respect to Ground | -1.0V to +7.0V |
| Maximum Operating Voltage | 6.6V |
| DC Output Current | 15.0mA |

CHAPTER 5

ZIGBEE

5.1 INTRODUCTION

ZigBee is the name of a specification for a suite of high level communication protocols using small, low-power digital radios based on the IEEE 802.15.4-2006 standard for wireless personal area networks (WPANs), such as wireless headphones connecting with cell phones via short range radio. The technology is intended to be simpler and less expensive than other WPAN’s, such as Bluetooth.

It is suitable for low data rate and low power consumption application with other wireless technologies such as Bluetooth and Wi-Fi. Application includes home and building automation, industrial control, vehicle management systems. The ZigBee module available in the regular 10m range version, and the 30m pro version.

5.2 WORKING PRINCIPLE

ZigBee hardware typically consist of an eight bit Microcontroller combined with a miniature transceiver a small amount (ex. 32 KB) of flash memory and RAM. The XBee module is interconnected with the eight bit microcontroller. The CMOS logic is used between the Microcontrolles amd XBEE module, the following figure explains how the information is transmitted and received using ZigBee.

5.3 TYPES OF ZIGBEE

ZigBee Coordinator (ZC)

The most capable device, the coordinator forms the root of the network tree and might bridge to other networks. There is exactly one Zigbee coordinator in each network since it is the device that started the network originally. It is able to store information about the network, acting as the Trust Centre.

ZigBee Router (ZR)

It can act as an intermediate router, passing data from other devices.

ZigBee End Devices (ZED)

Contains just enough functionality to talk to the parent node, it cannot relay data from other devices. This relationship allows the node to be asleep a significant amount of the time thereby giving long battery life. A ZED requires the least amount of memory, and can be less expensive to manufacture than a ZR or ZC.

5.4 AREAS OF APPLICATION

HOME AWARENESS: Water sensors, power sensors, smoke and fire detectors, smart appliances and access sensors.

MOBILE SERVICES: m-payement, m-monitoring and control, m-security and access control.

INDUSTRIAL PLANT: Process control, asset management, environmental management, industrial device control.

CHAPTER 6

BUZZER

6.1 INTRODUCTION

A buzzer is a signaling device, used in automobiles, household appliances such as microwave oven. The piezoelectric sound components operate on an principle utilizing natural oscillation of piezoelectric ceramics. Today, piezoelectric sound components are used in many ways such as home appliances, audio equipment telephones. They are applied widely in alarms, speakers, telephone ringers, receivers, transmitters, beep sounds, etc.

6.2 WORKING PRINCIPLE

Basically, the sound source of a piezoelectric sound component is a piezoelectric diaphragm. It consists of a piezoelectric ceramic plate which has electrodes on both sides and a metal plate. Ceramic plate is attached to a metal plate with adhesives. Applying D.C. voltage between electrodes of a piezoelectric diaphragm causes mechanical distortion due to the piezoelectric effect.

Diaphragm operation

For a misshaped piezoelectric element, the distortion of the piezoelectric element expands in a radial direction. The metal plate bonded to the piezoelectric element does not expand. Conversely , when the piezoelectric element shrinks, thus, when A.C. voltage is applied across electrodes, producing sound waves in the air.

CHAPTER 7

SCHEMATIC DIAGRAMS

7.1 ULTRASONIC SENSOR

7.2 ZIGBEE

7.3 MICROCONTROLLER

7.4 MAX3232EPE

7.4 LD1117S33 SOT-233

7.4 TIP122

CHAPTER 8

PROGRAMMING OF AT89C51

$MOD52    					; library file

ORG  	0000H

LJMP	MAIN

ORG   0100H

; DEFINITIONS

SPDAT		EQU	086H	;SPI Data Register

AUXR		EQU	08EH

SPSR		EQU	0AAH	;SPI Status Register

T2MOD		EQU	0C9H

SPCTL		EQU	0D5H	;SPI Control Register

SPACE		EQU	020H

;BIT MASKS

BIT0		EQU	001H

BIT1		EQU	002H

BIT2		EQU	004H

BIT3		EQU	008H

BIT4		EQU	010H

BIT5		EQU	020H

BIT6		EQU	040H

BIT7		EQU	080H

;SPI Control Register Bit Masks

SPIE		EQU	BIT7

SPE		EQU	BIT6

DORD		EQU	BIT5

MSTR		EQU	BIT4

CPOL		EQU	BIT3

CPHA		EQU	BIT2

SPR1		EQU	BIT1

SPR0		EQU	BIT0

;SPI Status Register Bit Masks

SPIF		EQU	BIT7

SPWCOL		EQU	BIT6

;Assumption: Normally DAC_CS is low, which will permit to write

;data into DAC_LOW and DAC_HIGH latches without affecting DAC output.

;Then, DAC_CS is pulled high and then down to give the digital input

;and DAC and allow further data to be written into DAC_LOW and DAC_HIGH

;latches

DAC_CS		EQU	BIT7

BUZZER1		EQU	BIT0

IR_TX1		EQU	BIT1

IR_TX2		EQU	BIT2

BUZZER2		EQU	BIT3

BUZZER1_ON_MASK		EQU	BIT0

BUZZER1_OFF_MASK	EQU	0FEH

IR_TX1_ON_MASK		EQU	BIT1

IR_TX1_OFF_MASK		EQU	0FDH

IR_TX2_ON_MASK		EQU	BIT2

IR_TX2_OFF_MASK		EQU	0FBH

IR_TX1_TX2_ON_MASK	EQU	006H

IR_TX1_TX2_OFF_MASK	EQU	0F9H

BUZZER2_ON_MASK		EQU	BIT3

BUZZER2_OFF_MASK	EQU	0F7H

IR_RX1_MASK	EQU	BIT2

IR_RX2_MASK	EQU	BIT3

; User defined registers in internal memory

STATUS		EQU	031H

CMD_RCVD	EQU	035H

SECOND_CNT	EQU	036H

DAC_LOW_DATA	EQU	037H

DAC_HIGH_DATA	EQU	038H

LED_REFRESH_CNT	EQU	039H

TEMP		EQU	03AH

TEST_FLAG	EQU	03BH

; Memory mapped I/O Register addresses

;Latches

DAC_LOW_LE	EQU	8000H

DAC_HIGH_LE	EQU	8001H

LCD1_LE		EQU	8000H

OUTPUT_1_LE	EQU	8100H

OUTPUT_2_LE	EQU	8200H

STEPPER1_LE	EQU	8300H

;Buffers

INPUT_1_OE	EQU	4000H

MAIN:

MOV 	TMOD, #20H

MOV	TH1, #0FDH   ; Baud rate set to 9600 for 11.0592 MHz crystal

MOV	SCON, #50H

MOV	PCON, #80H   ; Double baud rate 19200 for 11.0592 MHz crystal

MOV	T2CON, #00H	;Clear TCLK and RCLK in order to use timer1

MOV	TCON, #40H

MOV	AUXR,#01H	;Clearing EXTRAM(bit1) allows the expanded RAM of 768B (00H to 2FFH) to be

;indirectly accessible by MOVX.  Setting A0(bit0) allows ALE is active

;only during MOVX and MOVC

MOV	LED_REFRESH_CNT,#00H

MOV	TEST_FLAG,#00H

MOV	A,#00H	;Switch off Buzzer1 & 2, IR Tx1, and IR Tx2

MOV	DAC_HIGH_DATA,A

MOV	DAC_LOW_DATA,#00H

MOV	SPCTL,#053H	;Disable SPI interrupt, Enable SPI, MSB First, Master Mode, SCK active HIGH, shift triggered ;on leading edge, SCK=fosc/128

MOV	SPSR,#000H	;Clear SPIF, SPWCOL bits in the SPI Status Register

MOV	P0,#0FFH	;Write 1s into all port bits in order to make port0 as input

SETB	P3.2		;Write 1 into port3 pin2(IR receiver 1) in order to make it as input port

SETB	P3.3		;Write 1 into port3 pin3(IR receiver 2) in order to make it as input port

LCALL	STEP4		;By default, Gate is open

MOV	STATUS,#000H

LJMP	$

;Stepper Motor

;Clock Wise (CW) Rotation

;Step 1, 2, 3, 4

;CW_ROTATION:

;	LCALL	CW_CYCLE

;	LJMP	CW_ROTATION

;	RET

;Counter Clock Wise (CCW) Rotation

;Step 3, 2, 1, 4

;CCW_ROTATION:

;	LCALL	CCW_CYCLE

;	LJMP	CCW_ROTATION

;	RET

;Generate IR signal with continous 1's

;Pulse Rate: 38 KHz(13 microsecond on, 13 microsecond off), Burst Length = 20 cycles, Gap Length = 40 cycles

;GENERATE_CONTINOUS_IR_BIT_1:

;	LCALL	GENERATE_IR_BIT_1

;	MOV	A,P3

;	ANL	A,#IR_RX1_MASK

;	CJNE	A,#IR_RX1_MASK,IR_RX1_LOW

;IR_RX1_HIGH:

;	MOV	A,DAC_HIGH_DATA

;	ORL	A,#BUZZER1_ON_MASK

;	MOV	DAC_HIGH_DATA,A

;	LCALL	WRITE_DAC_DATA

;	LJMP	TO_GENERATE_IR_GAP1

;IR_RX1_LOW:

;	MOV	A,DAC_HIGH_DATA

;	ANL	A,#BUZZER1_OFF_MASK

;	MOV	DAC_HIGH_DATA,A

;	LCALL	WRITE_DAC_DATA

;TO_GENERATE_IR_GAP1:

;	LCALL	GENERATE_IR_GAP

;	LJMP	GENERATE_CONTINOUS_IR_BIT_1

MOV	STATUS,#000H

MOV	SECOND_CNT,#000H

MOV	CMD_RCVD,#000H

CHECK_SECOND_CNT:

MOV	A,SECOND_CNT

CJNE	A,#000H,RCV_CMD

LJMP	SEND_STATUS

RCV_CMD:

JNB	RI,CHECK_LOOP

MOV	A,SBUF

MOV	CMD_RCVD,A

CLR	RI

LCALL DELAY

LJMP	$

TO_CHECK_LOOP:

LJMP	CHECK_LOOP

INVALID_CMD_RCVD:

LCALL	SEND_QUOTE

LCALL	SEND_I

LCALL	SEND_N

LCALL	SEND_V

LCALL	SEND_A

LCALL	SEND_L

LCALL	SEND_I

LCALL	SEND_D

LCALL	SEND_SPACE

LCALL	SEND_C

LCALL	SEND_O

LCALL	SEND_M

LCALL	SEND_M

LCALL	SEND_A

LCALL	SEND_N

LCALL	SEND_D

LCALL	SEND_SPACE

LCALL	SEND_R

LCALL	SEND_E

LCALL	SEND_C

LCALL	SEND_E

LCALL	SEND_I

LCALL	SEND_V

LCALL	SEND_E

LCALL	SEND_D

LCALL	SEND_QUOTE

LCALL	SEND_NEW_LINE

LCALL	SEND_CARRIAGE_RETURN

LJMP	INCREMENT_SECOND_CNT

CHECK_LOOP:

LJMP	INCREMENT_SECOND_CNT

SEND_STATUS:

LCALL	SEND_QUOTE

LCALL	SEND_STATUS

LCALL	SEND_QUOTE

LCALL	SEND_NEW_LINE

LCALL	SEND_CARRIAGE_RETURN

INCREMENT_SECOND_CNT:

MOV	A,SECOND_CNT

ADD	A,#001H

MOV	SECOND_CNT,A

CJNE	A,#09H,END_INCREMENT_SECOND_CNT

MOV	SECOND_CNT,#000H

END_INCREMENT_SECOND_CNT:

LCALL	ONE_SECOND_DELAY

LJMP	CHECK_SECOND_CNT

GENERATE_IR_BIT_1:

;Generate 20 IR Pulses

MOV	P0,#0FFH

MOV	DPTR,#DAC_HIGH_LE

LCALL	GENERATE_IR_PULSE

LCALL	GENERATE_IR_PULSE

LCALL	GENERATE_IR_PULSE

LCALL	GENERATE_IR_PULSE

LCALL	GENERATE_IR_PULSE

LCALL	GENERATE_IR_PULSE

LCALL	GENERATE_IR_PULSE

LCALL	GENERATE_IR_PULSE

LCALL	GENERATE_IR_PULSE

LCALL	GENERATE_IR_PULSE

LCALL	GENERATE_IR_PULSE

LCALL	GENERATE_IR_PULSE

LCALL	GENERATE_IR_PULSE

LCALL	GENERATE_IR_PULSE

LCALL	GENERATE_IR_PULSE

LCALL	GENERATE_IR_PULSE

LCALL	GENERATE_IR_PULSE

LCALL	GENERATE_IR_PULSE

LCALL	GENERATE_IR_PULSE

LCALL	GENERATE_IR_PULSE

RET

GENERATE_IR_PULSE:

MOV	A,DAC_HIGH_DATA

ORL	A,#IR_TX1_ON_MASK

MOV	DAC_HIGH_DATA,A

MOVX	@DPTR,A

LCALL	THIRTEEN_MICROSECOND_DELAY

MOV	A,DAC_HIGH_DATA

ANL	A,#IR_TX1_OFF_MASK

MOV	DAC_HIGH_DATA,A

MOVX	@DPTR,A

LCALL	THIRTEEN_MICROSECOND_DELAY

RET

GENERATE_IR_GAP:

;Generate 40 IR Pulse Delays

LCALL	IR_PULSE_DELAY

LCALL	IR_PULSE_DELAY

LCALL	IR_PULSE_DELAY

LCALL	IR_PULSE_DELAY

LCALL	IR_PULSE_DELAY

LCALL	IR_PULSE_DELAY

LCALL	IR_PULSE_DELAY

LCALL	IR_PULSE_DELAY

LCALL	IR_PULSE_DELAY

LCALL	IR_PULSE_DELAY

LCALL	IR_PULSE_DELAY

LCALL	IR_PULSE_DELAY

LCALL	IR_PULSE_DELAY

LCALL	IR_PULSE_DELAY

LCALL	IR_PULSE_DELAY

LCALL	IR_PULSE_DELAY

LCALL	IR_PULSE_DELAY

LCALL	IR_PULSE_DELAY

LCALL	IR_PULSE_DELAY

LCALL	IR_PULSE_DELAY

LCALL	IR_PULSE_DELAY

LCALL	IR_PULSE_DELAY

LCALL	IR_PULSE_DELAY

LCALL	IR_PULSE_DELAY

LCALL	IR_PULSE_DELAY

LCALL	IR_PULSE_DELAY

LCALL	IR_PULSE_DELAY

LCALL	IR_PULSE_DELAY

LCALL	IR_PULSE_DELAY

LCALL	IR_PULSE_DELAY

LCALL	IR_PULSE_DELAY

LCALL	IR_PULSE_DELAY

LCALL	IR_PULSE_DELAY

LCALL	IR_PULSE_DELAY

LCALL	IR_PULSE_DELAY

LCALL	IR_PULSE_DELAY

LCALL	IR_PULSE_DELAY

LCALL	IR_PULSE_DELAY

LCALL	IR_PULSE_DELAY

LCALL	IR_PULSE_DELAY

RET

;Approximately gives 10 ms delay for 11.0596 MHz crystal

DELAY:   MOV	R1, #10

DELAY1:	 MOV	R2, #255

DELAY2:  NOP

NOP

DJNZ	R2, DELAY2

DJNZ	R1, DELAY1

RET

;Approximately gives 13 microsecond delay for 11.0596 MHz crystal

;TOTAL NUMBER OF INSTRUCTION CYCLE REQUIRED FOR 13 MICROSECOND

;DELAY IS 13 * 10-6 / (12 * 10-6 / 11.0596) = 13 * 11.0596 / 12 = 12

;2 CYCLES FOR LCALL, 8 CYCLES FOR NOP, ONE CYCLE FOR RET

;Reduce fOUR NOPs to compensate for fOUR instructions before calling thirteen microsecond delay

THIRTEEN_MICROSECOND_DELAY:

;	NOP

;	NOP

;	NOP

;	NOP

NOP

NOP

NOP

NOP

RET

IR_PULSE_DELAY:

;Added fOUR NOPs to compensate for the removal of fOUR NOPs in thirteen microsecond delay

;	NOP

;	NOP

NOP

NOP

LCALL	THIRTEEN_MICROSECOND_DELAY

;Added fOUR NOPs to compensate for the removal of fOUR NOPs in thirteen microsecond delay

;	NOP

;	NOP

NOP

NOP

LCALL	THIRTEEN_MICROSECOND_DELAY

RET

ONE_TENTH_SECOND_DELAY:

LCALL DELAY

LCALL DELAY

LCALL DELAY

LCALL DELAY

LCALL DELAY

LCALL DELAY

LCALL DELAY

LCALL DELAY

LCALL DELAY

LCALL DELAY

RET

QUARTER_SECOND_DELAY:

LCALL DELAY

LCALL DELAY

LCALL DELAY

LCALL DELAY

LCALL DELAY

LCALL DELAY

LCALL DELAY

LCALL DELAY

LCALL DELAY

LCALL DELAY

LCALL DELAY

LCALL DELAY

LCALL DELAY

LCALL DELAY

LCALL DELAY

LCALL DELAY

LCALL DELAY

LCALL DELAY

LCALL DELAY

LCALL DELAY

LCALL DELAY

LCALL DELAY

LCALL DELAY

LCALL DELAY

LCALL DELAY

RET

HALF_SECOND_DELAY:

LCALL DELAY

LCALL DELAY

LCALL DELAY

LCALL DELAY

LCALL DELAY

LCALL DELAY

LCALL DELAY

LCALL DELAY

LCALL DELAY

LCALL DELAY

LCALL DELAY

LCALL DELAY

LCALL DELAY

LCALL DELAY

LCALL DELAY

LCALL DELAY

LCALL DELAY

LCALL DELAY

LCALL DELAY

LCALL DELAY

LCALL DELAY

LCALL DELAY

LCALL DELAY

LCALL DELAY

LCALL DELAY

LCALL DELAY

LCALL DELAY

LCALL DELAY

LCALL DELAY

LCALL DELAY

LCALL DELAY

LCALL DELAY

LCALL DELAY

LCALL DELAY

LCALL DELAY

LCALL DELAY

LCALL DELAY

LCALL DELAY

LCALL DELAY

LCALL DELAY

LCALL DELAY

LCALL DELAY

LCALL DELAY

LCALL DELAY

LCALL DELAY

LCALL DELAY

LCALL DELAY

LCALL DELAY

LCALL DELAY

LCALL DELAY

RET

ONE_SECOND_DELAY:

LCALL	HALF_SECOND_DELAY

LCALL	HALF_SECOND_DELAY

RET

THREE_SECOND_DELAY:

LCALL	ONE_SECOND_DELAY

LCALL	ONE_SECOND_DELAY

LCALL	ONE_SECOND_DELAY

RET

SEVEN_SECOND_DELAY:

LCALL	ONE_SECOND_DELAY

LCALL	ONE_SECOND_DELAY

LCALL	ONE_SECOND_DELAY

LCALL	ONE_SECOND_DELAY

LCALL	ONE_SECOND_DELAY

LCALL	ONE_SECOND_DELAY

LCALL	ONE_SECOND_DELAY

RET

TEN_SECOND_DELAY:

LCALL	THREE_SECOND_DELAY

LCALL	SEVEN_SECOND_DELAY

RET

THIRTY_SECOND_DELAY:

LCALL	THREE_SECOND_DELAY

LCALL	SEVEN_SECOND_DELAY

LCALL	THREE_SECOND_DELAY

LCALL	SEVEN_SECOND_DELAY

LCALL	THREE_SECOND_DELAY

LCALL	SEVEN_SECOND_DELAY

RET

FORTY_SECOND_DELAY:

LCALL	THREE_SECOND_DELAY

LCALL	SEVEN_SECOND_DELAY

LCALL	THREE_SECOND_DELAY

LCALL	SEVEN_SECOND_DELAY

LCALL	THREE_SECOND_DELAY

LCALL	SEVEN_SECOND_DELAY

LCALL	THREE_SECOND_DELAY

LCALL	SEVEN_SECOND_DELAY

RET

WRITE_DAC_DATA:

MOV	P0,#0FFH

MOV	DPTR,#DAC_HIGH_LE

MOV	A,DAC_HIGH_DATA		;UPPER 4 BITS OF DAC DATA, DAC_CS=0

MOVX	@DPTR,A

MOV	P0,#0FFH

MOV	DPTR,#DAC_LOW_LE

MOV	A,DAC_LOW_DATA		;LOWER 8 BITS OF DAC DATA

MOVX	@DPTR,A

MOV	P0,#0FFH

MOV	DPTR,#DAC_HIGH_LE

MOV	A,DAC_HIGH_DATA		;UPPER 4 BITS OF DAC DATA

ORL	A,#DAC_CS		;DAC_CS=1

MOVX	@DPTR,A

MOV	P0,#0FFH

MOV	DPTR,#DAC_HIGH_LE

MOV	A,DAC_HIGH_DATA		;UPPER 4 BITS OF DAC DATA, DAC_CS=0

MOVX	@DPTR,A

RET

SEND_A:

MOV	A,#'A'

MOV	SBUF,A

SND_A:	JNB	TI,SND_A

CLR	TI

LCALL DELAY

RET

SEND_B:

MOV	A,#'B'

MOV	SBUF,A

SND_B:	JNB	TI,SND_B

CLR	TI

LCALL DELAY

RET

SEND_C:

MOV	A,#'C'

MOV	SBUF,A

SND_C:	JNB	TI,SND_C

CLR	TI

LCALL DELAY

RET

SEND_D:

MOV	A,#'D'

MOV	SBUF,A

SND_D:	JNB	TI,SND_D

CLR	TI

LCALL DELAY

RET

SEND_E:

MOV	A,#'E'

MOV	SBUF,A

SND_E:	JNB	TI,SND_E

CLR	TI

LCALL DELAY

RET

SEND_F:

MOV	A,#'F'

MOV	SBUF,A

SND_F:	JNB	TI,SND_F

CLR	TI

LCALL DELAY

RET

SEND_G:

MOV	A,#'G'

MOV	SBUF,A

SND_G:	JNB	TI,SND_G

CLR	TI

LCALL DELAY

RET

SEND_H:

MOV	A,#'H'

MOV	SBUF,A

SND_H:	JNB	TI,SND_H

CLR	TI

LCALL DELAY

RET

SEND_I:

MOV	A,#'I'

MOV	SBUF,A

SND_I:	JNB	TI,SND_I

CLR	TI

LCALL DELAY

RET

SEND_J:

MOV	A,#'J'

MOV	SBUF,A

SND_J:	JNB	TI,SND_J

CLR	TI

LCALL DELAY

RET

SEND_K:

MOV	A,#'K'

MOV	SBUF,A

SND_K:	JNB	TI,SND_K

CLR	TI

LCALL DELAY

RET

SEND_L:

MOV	A,#'L'

MOV	SBUF,A

SND_L:	JNB	TI,SND_L

CLR	TI

LCALL DELAY

RET

SEND_M:

MOV	A,#'M'

MOV	SBUF,A

SND_M:	JNB	TI,SND_M

CLR	TI

LCALL DELAY

RET

SEND_N:

MOV	A,#'N'

MOV	SBUF,A

SND_N:	JNB	TI,SND_N

CLR	TI

LCALL DELAY

RET

SEND_O:

MOV	A,#'O'

MOV	SBUF,A

SND_O:	JNB	TI,SND_O

CLR	TI

LCALL DELAY

RET

SEND_P:

MOV	A,#'P'

MOV	SBUF,A

SND_P:	JNB	TI,SND_P

CLR	TI

LCALL DELAY

RET

SEND_Q:

MOV	A,#'Q'

MOV	SBUF,A

SND_Q:	JNB	TI,SND_Q

CLR	TI

LCALL DELAY

RET

SEND_R:

MOV	A,#'R'

MOV	SBUF,A

SND_R:	JNB	TI,SND_R

CLR	TI

LCALL DELAY

RET

SEND_S:

MOV	A,#'S'

MOV	SBUF,A

SND_S:	JNB	TI,SND_S

CLR	TI

LCALL DELAY

RET

SEND_T:

MOV	A,#'T'

MOV	SBUF,A

SND_T:	JNB	TI,SND_T

CLR	TI

LCALL DELAY

RET

SEND_U:

MOV	A,#'U'

MOV	SBUF,A

SND_U:	JNB	TI,SND_U

CLR	TI

LCALL DELAY

RET

SEND_V:

MOV	A,#'V'

MOV	SBUF,A

SND_V:	JNB	TI,SND_V

CLR	TI

LCALL DELAY

RET

SEND_W:

MOV	A,#'W'

MOV	SBUF,A

SND_W:	JNB	TI,SND_W

CLR	TI

LCALL DELAY

RET

SEND_X:

MOV	A,#'X'

MOV	SBUF,A

SND_X:	JNB	TI,SND_X

CLR	TI

LCALL DELAY

RET

SEND_Y:

MOV	A,#'Y'

MOV	SBUF,A

SND_Y:	JNB	TI,SND_Y

CLR	TI

LCALL DELAY

RET

SEND_Z:

MOV	A,#'Z'

MOV	SBUF,A

SND_Z:	JNB	TI,SND_Z

CLR	TI

LCALL DELAY

RET

SEND_0:

MOV	A,#'0'

MOV	SBUF,A

SND_0:	JNB	TI,SND_0

CLR	TI

LCALL DELAY

RET

SEND_1:

MOV	A,#'1'

MOV	SBUF,A

SND_1:	JNB	TI,SND_1

CLR	TI

LCALL DELAY

RET

SEND_2:

MOV	A,#'2'

MOV	SBUF,A

SND_2:	JNB	TI,SND_2

CLR	TI

LCALL DELAY

RET

SEND_3:

MOV	A,#'3'

MOV	SBUF,A

SND_3:	JNB	TI,SND_3

CLR	TI

LCALL DELAY

RET

SEND_4:

MOV	A,#'4'

MOV	SBUF,A

SND_4:	JNB	TI,SND_4

CLR	TI

LCALL DELAY

RET

SEND_5:

MOV	A,#'5'

MOV	SBUF,A

SND_5:	JNB	TI,SND_5

CLR	TI

LCALL DELAY

RET

SEND_6:

MOV	A,#'6'

MOV	SBUF,A

SND_6:	JNB	TI,SND_6

CLR	TI

LCALL DELAY

RET

SEND_7:

MOV	A,#'7'

MOV	SBUF,A

SND_7:	JNB	TI,SND_7

CLR	TI

LCALL DELAY

RET

SEND_8:

MOV	A,#'8'

MOV	SBUF,A

SND_8:	JNB	TI,SND_8

CLR	TI

LCALL DELAY

RET

SEND_9:

MOV	A,#'9'

MOV	SBUF,A

SND_9:	JNB	TI,SND_9

CLR	TI

LCALL DELAY

RET

SEND_QUOTE:

MOV	A,#'"'

MOV	SBUF,A

SND_QUOTE:	JNB	TI,SND_QUOTE

CLR	TI

LCALL DELAY

RET

SEND_COMMA:

MOV	A,#','

MOV	SBUF,A

SND_COMMA:	JNB	TI,SND_COMMA

CLR	TI

LCALL DELAY

RET

SEND_COLLON:

MOV	A,#':'

MOV	SBUF,A

SND_COLLON:	JNB	TI,SND_COLLON

CLR	TI

LCALL DELAY

RET

SEND_SPACE:

MOV	A,#' '

MOV	SBUF,A

SND_SPACE:	JNB	TI,SND_SPACE

CLR	TI

LCALL DELAY

RET

SEND_NEW_LINE:

MOV	A,#00BH

MOV	SBUF,A

SND_NEW_LINE:	JNB	TI,SND_NEW_LINE

CLR	TI

LCALL DELAY

RET

SEND_CARRIAGE_RETURN:

MOV	A,#00DH

MOV	SBUF,A

SND_CARRIAGE_RETURN:	JNB	TI,SND_CARRIAGE_RETURN

CLR	TI

LCALL DELAY

RET

CW_CYCLE:

LCALL	ONE_TENTH_SECOND_DELAY

LCALL	STEP1

LCALL	ONE_TENTH_SECOND_DELAY

LCALL	STEP2

LCALL	ONE_TENTH_SECOND_DELAY

LCALL	STEP3

LCALL	ONE_TENTH_SECOND_DELAY

LCALL	STEP4

RET

CCW_CYCLE:

LCALL	ONE_TENTH_SECOND_DELAY

LCALL	STEP3

LCALL	ONE_TENTH_SECOND_DELAY

LCALL	STEP2

LCALL	ONE_TENTH_SECOND_DELAY

LCALL	STEP1

LCALL	ONE_TENTH_SECOND_DELAY

LCALL	STEP4

RET

;Set Step1

STEP1:

MOV	A,#01EH

MOV	DAC_LOW_DATA,A

LCALL	WRITE_DAC_DATA

RET

;Set Step2

STEP2:

MOV	A,#36H

MOV	DAC_LOW_DATA,A

LCALL	WRITE_DAC_DATA

RET

;Set Step3

STEP3:

MOV	A,#33H

MOV	DAC_LOW_DATA,A

LCALL	WRITE_DAC_DATA

RET

;Set Step4

STEP4:

MOV	A,#1BH

MOV	DAC_LOW_DATA,A

LCALL	WRITE_DAC_DATA

RET

END

CHAPTER 9

DATA SHEETS

9.1 ULTRASONIC SENSOR

9.2 MICROCONTROLLER

9.3 ZIGBEE

CHAPTER 10

REFERENCES

- Muhammad Rashid.H, “ Power Electronics circuits, Devices and Application” , Eastern Economy Edition  Prentice Hall of India Private Limited.

- Charulatha Publication, “ Microprocessor And Microcontroller”

Online sources

- http://www.eaton.com

- http://www.senscomp.com