P.W.M. BASED SPEED

CONTROLLER FOR DC MOTOR

CONTENTS

# INTERODUCTION

Introduction of project

# Block Diagram

# Working and sections of project

Section list

Component used

Circuit Diagram

# Description

Power supply

Microcontroller

LCD and its Interface

LCD (liquid crystal diode)

LED (light emitting diode)

# Uses

# Scope and conclusion

6      Bibliography

INTRODUCTION

# Motor controller

A motor controller is a device or group of devices that serves to govern in some predetermined manner the performance of an electric motor. A motor controller might include a manual or automatic means for starting and stopping the motor, selecting forward or reverse rotation, selecting and regulating the speed, regulating or limiting the torque, and protecting against overloads and faults.

## Types of motor controllers

Motor controllers can be manually, remotely or automatically operated. They may include only the means for starting and stopping the motor or they may include other functions

An electric motor controller can be classified by the type of motor it is to drive such as permanent, servo, series, separately excited, and alternating current.

A motor controller is connected to a power source such as a battery pack or power supply, and control circuitry in the form of analog or digital input signals.

Adjustable-speed drives

An adjustable-speed drive (ASD) or variable-speed drive (VSD) is an interconnected combination of equipment that provides a means of driving and adjusting the operating speed of a mechanical load. An electrical adjustable-speed drive consists of an electric motor and a speed controller or power converter plus auxiliary devices and equipment. In common usage, the term “drive” is often applied to just the controller.

Speed Control of D.C. Series Motors :

Speed control of D.C. series motor may be accomplished through either

armature control, field control or series - parallel control.

1. ARMATURE CONTROL METHODS:  Speed adjustment of D.C.

series motors by armature control may be had by any one of the following

methods:

(a) ARMATURE RESISTANCE CONTROL: This is the most common

method employed. It is obtained in the same way as D.C. shunt motor with the exception that the control resistance may be connected directly in series with the supply to the complete motor. the drawbacks of armature resistance method for machines with shunt fields are not as important in the speed control of D.C. series motor. This method of speed control is most economical for constant torque drives and is employed chiefly for D.C. series motors driving cranes ,hoists, trains etc. because such devices operate on intermittent duty. the power lodd in the control resistance for many applications of the series d.c. motors is not too serious , since in these applications the control is utilized for a larger portion of time for reducing the speed under light load conditions and is used intermittently when the motor is carrying full-load.The maximum range of speed controlof about 3:1 will be available depending on the load.

(b) 	SHUNTED ARMATURE CONTROL: 	The combination of a

rheostat shunting the armature and a rheostat in series with the armature is used to give slow speeds at light loads.such a scheme accomplishes the speed control both by lowering the voltage applied to the motor armature and by varying the flux. The voltage applied to the armature terminals is varied by varying  series rheostat R1. The exciting current can be varied by varying the armature shunting resistance R2 for same armature current Ia .                                               For a given constant load torque , if armature Ia is reduced due to armature divertor then the flux must increase  because torque developed by the armature T is proportional to product of

flux and armature current. this causes increase in current drawn from the supply mains, so increase in flux and decrease in speed.

In this method speed control can be  obtained over a wide range but below normal speed. The limit of speed control range depends upon ratio existing between resistances R1and R2 and also on the current at which there is saturation of the magnetic circuit. No speed can effectively be adjusted to any desired low speed .This method is not economical due to

considerable power losses in the speed controlling resistances.

(c) ARMATURE TERMINAL VOLTAGE CONTROL: The speed control

of d.c. series motor can be accomplished by supplying the power to the motor from a separate variable voltage supply. this method is seldom used because of high cost of control equipment.

2. FIELD CONTROL METHODS:

The speed of d.c. series motor can be controlled by varying the flux in any

of the following manners:

(a) FIELD DIVERTOR METHOD: The field flux can be reduced by shunting  a portion of motor current around the series field thus reducing the excitation mmf and weakening of field. this method gives speed above normal because flux is reduced by this method. Lesser the divertor resistance, less the field current,less speed and hence more speed. This method is convenient as well as economical and provides the speed control range usual;y not exceeding 2:1.This method is used in drives in which the speed should rise sharply as soon as the load falls.

(b) TAPPED FIELD CONTROL: This is an another method of increasing the speed by reducing flux and is accomplished by reducing the number of turns of the field winding thgrough which the current flows. In this method a number of tappings form the field winding are brought outside. A number of series field turns can be shotr-circuited according to the requirement. When all field turns are in circuit , the motor runs at its lowest speedand speed increases with cutting out some of the sreies field turns. This method is usually employed in electric traction.

(c) PARALLELING FIELD COILS METHOD: In this method , used in fan motors several speeds can be obtained by regrouping of field coils.

MCU UNIT

KEYPAD

POWER SUPPLY

ISOLATION CKT

LCD

DC

MOTOR

POWER AMP

BLOCK DIAGRAM OF P.W.M. BASED SPEED CONTROLLER FOR DC MOTOR

WORKING

P.W.M. BASED SPEED CONTROLLLER FOR DC MOTOR

P.W.M. BASED DC MOTOR SPEED CONTROLLER CONSIST OF VARIOUS SECTION AND ARE GIVEN BELOW AS:-

- CIRCUIT DIAGRAM

- POWER SUPPLY

- MICROCONTROLLER UNIT

- LCD

- ISOLATION CIRCUIT

- KEYPAD

- POWER AMPLIFIER

- DC MOTOR

COMPONENTS USED

## Transformer

## Diode

## Electrolyte Capacitor

## 1000 micro farad

## 10 micro farad

## Regulated IC L17305CV

## Microcontrller AT89C52

## Transistor Tr 369

## Crystal Osc. 11.0592

## Resistor 470 ohm or 4.7 k ohm

## IC 4N35

## LED (light emitting Diode)

## LCD

## Toggel Switch

PO

POWER SUPPLY DESCRIPTION:

# TRANSFORMER

SHUNT CAPACITOR

BRIDGE RECTIFIER

VOLTAGE REGULATOR

The power supply circuit comprises of four basic parts:

The transformer steps down the 220 V a/c. into 12 V a/c. The transformer work on the principle of magnetic induction, where two coils: primary and secondary are wound around an iron core. The two coils are physically insulated from each other in such a way that passing an a/c. current through the primary coil creates a changing voltage in the primary coil and a changing magnetic field in the core. This in turn induces a varying a/c. voltage in the secondary coil.

The a/c. voltage is then fed to the bridge rectifier. The rectifier circuit is used in most electronic power supplies is the single-phase bridge rectifier with capacitor filtering, usually followed by a linear voltage regulator. A rectifier circuit is necessary to convert a signal having zero average value into a non-zero average value. A rectifier transforms alternating current into direct current by limiting or regulating the direction of flow of current. The output resulting from a rectifier is a pulsating D.C. voltage. This voltage is not appropriate for the components that are going to work through it.

1N4007								12-0-12 V																														1000uF																TRANSFORMER

7812

7805

The ripple of the D.C. voltage is smoothened using a filter capacitor of 1000 microF 25V. The filter capacitor stores electrical charge. If it is large enough the capacitor will store charge as the voltage rises and give up the charge as the voltage falls. This has the effect of smoothing out the waveform and provides steadier voltage output. A filter capacitor is connected at the rectifier output and the d.c voltage is obtained across the capacitor. When this capacitor is used in this project, it should be twice the supply voltage. When the filter is used, the RC charge time of the filter capacitor must be short and the RC discharge time must be long to eliminate ripple action. In other words the capacitor must charge up fast, preferably with no discharge.

- When the rectifier output voltage is increasing, the capacitor charges to the peak voltage Vm. Just past the positive peak, the rectifier output voltage starts to fall but at this point the capacitor has +Vm voltage across it. Since the source voltage becomes slightly less than Vm, the capacitor will try to send current back through the diode of rectifier. This reverse biases the diode. The diode disconnects or separates the source the source form load. The capacitor starts to discharge through load. This prevents the load voltage from falling to zero. The capacitor continues to discharge until source voltage becomes more than capacitor voltage. The diode again starts conducting and the capacitor is again charged to peak value Vm. When capacitor is charging the rectifier supplies the charging through capacitor branch as well as load current, the capacitor sends currents through the load. The rate at which capacitor discharge depends upon time constant RC. The longer the time constant, the steadier is the output voltage. An increase in load current i.e. decrease in resistance makes time constant of discharge path smaller. The ripple increase and d.c output voltage V dc decreases. Maximum capacity cannot exceed a certain limit because the larger the capacitance the greater is the current required to charge the capacitor.

The voltage regulator regulates the supply if the supply if the line voltage increases or decreases. The series 78xx regulators provide fixed regulated voltages from 5 to 24 volts. An unregulated input voltage is applied at the IC Input pin i.e. pin 1 which is filtered by capacitor. The out terminal of the IC i.e. pin 3 provides a regular output.  The third terminal is connected to ground. While the input voltage may vary over some permissible voltage range, and the output voltage remains constant within specified voltage variation limit. The 78xx IC’s are positive voltage regulators whereas 79xx IC’s are negative voltage regulators.

These voltage regulators are integrated circuits designed as fixed voltage regulators for a wide variety of applications. These regulators employ current limiting, thermal shutdown and safe area compensation. With adequate heat sinking they can deliver output currents in excess of 1 A. These regulators have internal thermal overload protection. It uses output transistor safe area compensation and the output voltage offered is in 2% and 4% tolerance.

THE MICROCONTROLLER

ER:

In our day to day life the role of micro-controllers has been immense. They are used in a variety of applications ranging from home appliances, FAX machines, Video games,Camera, Exercise equipment, Cellular phones musical Instruments to Computers, engine control, aeronautics, security systems and the list goes on.

MICROCONTROLLERS VERSUS MICROPROCESSORS

What is the difference between a microprocessor and microcontroller? The microprocessors (such as 8086,80286,68000 etc.) contain no RAM, no ROM and no I/O ports on the chip itself. For this reason they are referred as general- purpose microprocessors. A system designer using general- purpose microprocessor must add external RAM, ROM, I/O ports and timers to make them functional. Although the addition of external RAM, ROM, and I/O ports make the system bulkier and much more expensive, they have the advantage of versatility such that the designer can decide on the amount of RAM, ROM and I/o ports needed to fit the task at hand. This is the not the case with microcontrollers. A microcontroller has a CPU (a microprocessor) in addition to the fixed amount of RAM, ROM, I/O ports, and timer are all embedded together on the chip: therefore, the designer cannot add any external memory, I/O, or timer to it. The fixed amount of on chip RAM, ROM, and number of I/O ports in microcontrollers make them ideal for many applications in which cost and space are critical. In many applications, for example a TV remote control, there is no need for the computing power of a 486 or even a 8086 microprocessor. In many applications, the space it takes, the power it consumes, and the price per unit are much more critical considerations than the computing power. These applications most often require some I/O operations to read signals and turn on and off certain bits. It is interesting to know that some microcontrollers manufactures have gone as far as integrating an ADC and other peripherals into the microcontrollers.

EXTERNAL

INTERRUPTS

ON-CHIP ROM for program code

ON-CHIP RAM

ETC.

TIMER 0

TIMER 1

INTERRUPT CONTROL

### CPU

SERIAL PORT

## 4 I/O

PORTS

BUS CONTROL

### OSC

TXD  RXD

MICROCONTROLLER BLOCK DIAGRAM

MICROCONTROLLERS FOR EMBEDDED SYSTEMS

In the literature discussing microprocessors, we often see a term embedded system. Microprocessors and microcontrollers are widely used in embedded system products. An embedded product uses a microprocessor (or microcontroller) to do one task and one task only. A printer is an example of embedded system since the processor inside it performs one task only: namely, get data and print it. Contrasting this with a IBM PC which can be used for a number of applications such as word processor, print server, network server, video game player, or internet terminal. Software for a variety of applications can be loaded and run. Of course the reason a PC can perform myriad tasks is that it has RAM memory and an operating system that loads the application software into RAM and lets the CPU run it. In an embedded system, there is only one application software that is burned into ROM. An PC contains or is connected to various embedded products such as the keyboard, printer, modem, disk controller, sound card, CD-ROM driver, mouse and so on. Each one of these peripherals has a microcontroller inside it that performs only one task. For example, inside every mouse there is a microcontroller to perform the task of finding the mouse position and sending it to the PC.

Although microcontrollers are the preferred choice for many embedded systems, there are times that a microcontroller is inadequate for the task. For this reason, in many years the manufacturers for general-purpose microprocessors have targeted their microprocessor for the high end of the embedded market.

INTRODUCTION TO 8051

In 1981, Intel Corporation introduced an 8-bit microcontroller called the 8051. This microcontroller had 128 bytes of RAM, 4K bytes of on-chip ROM, two timers, one serial port, and four ports (8-bit) all on a single chip. The 8051 is an 8-bit processor, meaning the CPU can work on only 8- bit pieces to be processed by the CPU. The 8051 has a total of four I/O ports, each 8- bit wide. Although 8051 can have a maximum of 64K bytes of on-chip ROM, many manufacturers put only 4K bytes on the chip.

The 8051 became widely popular after Intel allowed other manufacturers to make any flavor of the 8051 they please with the condition that they remain code compatible with the 8051. This has led to many versions of the 8051 with different speeds and amount of on-chip ROM marketed by more than half a dozen manufacturers. It is important to know that although there are different flavors of the 8051, they are all compatible with the original 8051 as far as the instructions are concerned. This means that if you write your program for one, it will run on any one of them regardless of the manufacturer. The major 8051 manufacturers are Intel, Atmel, Dallas Semiconductors, Philips Corporation, Infineon.

AT89C51 FROM ATMEL CORPORATION

This popular 8051 chip has on-chip ROM in the form of flash memory. This is ideal for fast development since flash memory can be erased in seconds compared to twenty minutes or more needed for the earlier versions of the 8051. To use the AT89C51 to develop a microcontroller-based system requires a ROM burner that supports flash memory: However, a ROM eraser is not needed. Notice that in flash memory you must erase the entire contents of ROM in order to program it again. The PROM burner does this erasing of flash itself and this is why a separate burner is not needed. To eliminate the need for a PROM burner Atmel is working on a version of the AT89C51 that can be programmed by the serial COM port of the PC.

FEATURES OF AT89C51

- 4K on-chip ROM

- 128 bytes internal RAM (8-bit)

- 32 I/O pins

- Two 16-bit timers

- Six Interrupts

- Serial programming facility

- 40 pin Dual-in-line Package

PIN DESCRIPTION

The 89C51 have a total of 40 pins that are dedicated for various functions such as I/O, RD, WR, address and interrupts. Out of 40 pins, a total of 32 pins are set aside for the four ports P0, P1, P2, and P3, where each port takes 8 pins. The rest of the pins are designated as Vcc, GND, XTAL1, XTAL, RST, EA, and PSEN. All these pins except PSEN and ALE are used by all members of the 8051 and 8031 families. In other words, they must be connected in order for the system to work, regardless of whether the microcontroller is of the 8051 or the 8031 family. The other two pins, PSEN and ALE are used mainly in 8031 based systems.

Vcc

Pin 40 provides supply voltage to the chip. The voltage source is +5 V.

GND

Pin 20 is the ground.

XTAL1 and XTAL2

The 8051 have an on-chip oscillator but requires external clock to run it. Most often a quartz crystal oscillator is connected to input XTAL1 (pin 19) and XTAL2 (pin 18). The quartz crystal oscillator connected to XTAL1 and XTAL2 also needs two capacitors of 30 pF value. One side of each capacitor is connected to the ground.

C2

XTAL2

C1

XTAL1

GND

It must be noted that there are various speeds of the 8051 family. Speed refers to the maximum oscillator frequency connected to the XTAL. For example, a 12 MHz chip must be connected to a crystal with 12 MHz frequency or less. Likewise, a 20 MHz microcontroller requires a crystal frequency of no more than 20 MHz. When the 8051 is connected to a crystal oscillator and is powered up, we can observe the frequency on the XTAL2 pin using oscilloscope.

RST

Pin 9 is the reset pin. It is an input and is active high (normally low). Upon applying a high pulse to this pin, the microcontroller will reset and terminate all activities. This is often referred to as a power –on reset. Activating a power-on reset will cause all values in the registers to be lost. Notice that the value of Program Counter is 0000 upon reset, forcing the CPU to fetch the first code from ROM memory location 0000. This means that we must place the first line of source code in ROM location 0000 that is where the CPU wakes up and expects to find the first instruction. In order to RESET input to be effective, it must have a minimum duration of 2 machine cycles. In other words, the high pulse must be high for a minimum of 2 machine cycles before it is allowed to go low.

EA

All the 8051 family members come with on-chip ROM to store programs. In such cases, the EA pin is connected to the Vcc. For family members such as 8031 and 8032 in which there is no on-chip ROM, code is stored on an external ROM and is fetched by the 8031/32. Therefore for the 8031 the EA pin must be connected to ground to indicate that the code is stored externally. EA, which stands for “external access,” is pin number 31 in the DIP packages. It is input pin and must be connected to either Vcc or GND. In other words, it cannot be left unconnected.

PSEN

This is an output pin. PSEN stands for “program store enable.” It is the read strobe to external program memory. When the microcontroller is executing from external memory, PSEN is activated twice each machine cycle.

ALE

ALE (Address latch enable) is an output pin and is active high. When connecting a microcontroller to external memory, potr 0 provides both address and data. In other words the microcontroller multiplexes address and data through port 0 to save pins. The ALE pin is used for de-multiplexing the address and data by connecting to the G pin of the 74LS373 chip.

I/O port pins and their functions

The four ports P0, P1, P2, and P3 each use 8 pins, making them 8-bit ports. All the ports upon RESET are configured as output, ready to be used as output ports. To use any of these as input port, it must be programmed.

Port 0

Port 0 occupies a total of 8 pins (pins 32 to 39). It can be used for input or output. To use the pins of port 0 as both input and output ports, each pin must be connected externally to a 10K-ohm pull-up resistor. This is due to fact that port 0 is an open drain, unlike P1, P2 and P3. With external pull-up resistors connected upon reset, port 0 is configured as output port. In order to make port 0 an input, the port must be programmed by writing 1 to all the bits of it. Port 0 is also designated as AD0-AD7, allowing it to be used for both data and address. When connecting a microcontroller to an external memory, port 0 provides both address and data. The microcontroller multiplexes address and data through port 0 to save pins. ALE indicates if P0 has address or data. When ALE=0, it provides data D0-D7, but when ALE=1 it has address A0-A7. Therefore, ALE is used for de-multiplexing address and data with the help of latch 74LS373.

Port 1

Port 1 occupies a total of 8 pins (pins 1 to 8). It can be used as input or output. In contrast to port 0, this port does not require pull-up resistors since it has already pull-up resistors internally. Upon reset, port 1 is configures as an output port. Similar to port 0, port 1 can be used as an input port by writing 1 to all its bits.

Port 2

Port 2 occupies a total of 8 pins (pins 21 to 28). It can be used as input or output. Just like P1, port 2 does not need any pull-up resistors since it has pull-up resistors internally. Upon reset port 2 is configured as output port. To make port 2 input, it must be programmed as such by writing 1s to it.

Port 3

Port 3 occupies a total of 8 pins (pins 10 to 17). It can be used as input or output. P3 does not need any pull-up resistors, the same as P1 and P2 did not. Although port 3 is configured as output port upon reset, this is not the way it is most commonly used. Port 3 has an additional function of providing some extremely important signals such as interrupts. Some of the alternate functions of P3 are listed below:

P3.0      RXD (Serial input)

P3.1      TXD (Serial output)

P3.2      INT0 (External interrupt 0)

P3.3      INT1 (External interrupt 1)

P3.4      T0 (Timer 0 external input)

P3.5      T1 (Timer 1 external input)

P3.6      WR (External memory write strobe)

P3.7      RD (External memory read strobe)

INSIDE THE 89C51

Registers

In the CPU, registers are used to store information temporarily. That information could be a byte of data to be processed, or an address pointing to the data to be fetched. In the 8051 there us only one data type: 8 bits. With an 8- bit data type, any data larger than 8 bits has to be broken into 8-bit chunks before it is processed.

## A

## B

R0

R1

R2

R3

R4

R5

R6

R7

## DPH                 DPL

DPTR

## Program Counter

PC

(b) Some 8051 16-bit registers

- Some 8051 8-bit registers

The most commonly used registers of the 8051 are A(accumulator), B, R0, R1, R2, R3, R4, R5, R6, R7, DPTR (data pointer) and PC (program counter). All the above registers are 8-bit registers except DPTR and the program counter. The accumulator A is used for all arithmetic and logic instructions.

Program Counter and Data Pointer

The program counter is a 16- bit register and it points to the address of the next instruction to be executed. As the CPU fetches op-code from the program ROM, the program counter is incremented to point to the next instruction. Since the PC is 16 bit wide, it can access program addresses 0000 to FFFFH, a total of 64K bytes of code. However, not all the members of the 8051 have the entire 64K bytes of on-chip ROM installed.

The DPTR register is made up of two 8-bit registers, DPH and DPL, which are used to furnish memory addresses for internal and external data access. The DPTR is under the control of program instructions and can be specified by its name, DPTR. DPTR does not have a single internal address, DPH and DPL are assigned an address each.

Flag bits and the PSW Register

Like any other microprocessor, the 8051 have a flag register to indicate arithmetic conditions such as the carry bit. The flag register in the 8051 is called the program status word (PSW) register.

The program status word (PSW) register is an 8-bit register. It is also referred as the flag register. Although the PSW register is 8-bit wide, only 6 bits of it are used by the microcontroller. The two unused bits are user definable flags. Four of the flags are conditional flags, meaning they indicate some conditions that resulted after an instruction was executed. These four are CY (carry), AC (auxiliary carry), P (parity), and OV (overflow). The bits of the PSW register are shown below:

## CY       AC       F0       RS1       RS0       OV       --         P

CY     PSW.7          Carry flag

AC     PSW.6          Auxiliary carry flag

--        PSW.5          Available to the user for general purpose

RS1   PSW.6          Register bank selector bit 1

RS0   PSW.3          Register bank selector bit 0

OV     PSW.2          Overflow flag

F0      PSW.1          User definable bit

P        PSW.0          Parity flag

CY, the carry flag

This flag is set whenever there is a carry out from the d7 bit. This flag bit is affected after an 8-bit addition or subtraction. It can also be set to 1 or 0 directly by an instruction such as “SETB C” and “CLR C” where “SETB C” stands for set bit carry and “CLR C” for clear carry.

AC, the auxiliary carry flag

If there is carry from D3 to D4 during an ADD or SUB operation, this bit is set: otherwise cleared. This flag is used by instructions that perform BCD arithmetic.

P, the parity flag

The parity flag reflects the number of 1s in the accumulator register only. If the register A contains an odd number of 1s, then P=1. Therefore, P=0 if Ahas an even number of 1s.

OV, the overflow flag

This flag is set whenever the result of a signed number operation is too large, causing the high order bit to overflow into the sign bit. In general the carry flags is used to detect errors in unsigned arithmetic operations.

MEMORY SPACE ALLOCATION

- Internal ROM    The 89C51 has a 4K bytes of on-chip ROM. This 4K bytes ROM memory has memory addresses of 0000 to 0FFFh. Program addresses higher than 0FFFh, which exceed the internal ROM capacity will cause the microcontroller to automatically fetch code bytes from external memory. Code bytes can also be fetched exclusively from an external memory, addresses 0000h to FFFFh, by connecting the external access pin to ground. The program counter doesn’t care where the code is: the circuit designer decides whether the code is found totally in internal ROM, totally in external ROM or in a combination of internal and external ROM.

- Internal RAM

The 1289 bytes of RAM inside the 8051 are assigned addresses 00 to 7Fh. These 128 bytes can be divided into three different groups as follows:

- A total of 32 bytes from locations 00 to 1Fh are set aside for register banks and the stack.

- A total of 16 bytes from locations 20h to 2Fh are set aside for bit addressable read/write memory and instructions.

### Liquid Crystal Display

#### 3.2.12.1 LCD Display

Liquid crystal displays (LCD) are widely used in recent years as compares to LEDs. This is due to the declining prices of LCD, the ability to display numbers, characters and graphics, incorporation of a refreshing controller into the LCD, their by relieving the CPU of the task of refreshing the LCD and also the ease of programming for characters and graphics. HD 44780 based LCDs are most commonly used.

LCD pin description

The LCD discuss in this section has the most common connector used for the Hitatchi 44780 based LCD is 14 pins in a row and modes of operation and how to program and interface with microcontroller is describes in this section.

Fig 3.21 LCD Pin Description Diagram

VCC, VSS, VEE

The voltage VCC and VSS provided by +5V and ground respectively while VEE is used for controlling LCD contrast.  Variable voltage between Ground and Vcc is used to specify the contrast (or "darkness") of the characters on the LCD screen.

RS (register select)

There are two important registers inside the LCD. The RS pin is used for their selection as follows. If  RS=0, the instruction command code register is selected, then allowing to user to send a command such as clear display, cursor at home etc.. If RS=1, the data register is selected, allowing the user to send data to be displayed on the LCD.

R/W (read/write)

The R/W (read/write) input allowing the user to write information from it. R/W=1, when it read and R/W=0, when it writing.

EN (enable)

The enable pin is used by the LCD to latch information presented to its data pins. When data is supplied to data pins, a high power, a high-to-low pulse must be applied to this pin in order to for the LCD to latch in the data presented at the data pins.

D0-D7 (data lines)

The 8-bit data pins, D0-D7, are used to send information to the LCD or read the contents of the LCD’s internal registers. To displays the letters and numbers, we send ASCII codes for the letters A-Z, a-z, and numbers 0-9 to these pins while making RS =1. There are also command codes that can be sent to clear the display or force the cursor to the home position or blink the cursor.

We also use RS =0 to check the busy flag bit to see if the LCD is ready to receive the information. The busy flag is D7 and can be read when R/W =1 and RS =0, as follows: if R/W =1 and RS =0, when D7 =1(busy flag =1), the LCD is busy taking care of internal operations and will not accept any information. When D7 =0, the LCD is ready to receive new information.

| Pin | Symbol | I/O | Description |
| --- | --- | --- | --- |
| 1 | VSS | - | Ground |
| 2 | VCC | - | +5V power supply |
| 3 | VEE | - | Power supply to control contrast |
| 4 | RS | I | RS=0 to select command register, RS=1 to select data register. |
| 5 | R/W | I | R/W=0 for write, R/W=1 for read |
| 6 | E | I/O | Enable |
| 7 | PB0 | I/O | The 8 bit data bus |
| 8 | PB1 | I/O | The 8 bit data bus |
| 9 | DB2 | I/O | The 8 bit data bus |
| 10 | DB3 | I/O | The 8 bit data bus |
| 11 | DB4 | I/O | The 8 bit data bus |
| 12 | DB5 | I/O | The 8 bit data bus |
| 13 | DB6 | I/O | The 8 bit data bus |
| 14 | DB7 | I/O | The 8 bit data bus |

### LCD Backgorund

Frequently, an 8051 program must interact with the outside world using input and output devices that communicate directly with a human being. One of the most common devices attached to an 8051 is an LCD display. Some of the most common LCDs connected to the 8051 are 16x2 and 20x2 displays. This means 16 characters per line by 2 lines and 20 characters per line by 2 lines, respectively.

Fortunately, a very popular standard exists which allows us to communicate with the vast majority of LCDs regardless of their manufacturer. The standard is referred to as HD44780U, which refers to the controller chip which receives data from an external source (in this case, the 8051) and communicates directly with the LCD.

### 44780 BACKGROUND

The 44780 standard requires 3 control lines as well as either 4 or 8 I/O lines for the data bus. The user may select whether the LCD is to operate with a 4-bit data bus or an 8-bit data bus. If a 4-bit data bus is used the LCD will require a total of 7 data lines (3 control lines plus the 4 lines for the data bus). If an 8-bit data bus is used the LCD will require a total of 11 data lines (3 control lines plus the 8 lines for the data bus).

The three control lines are referred to as EN, RS, and RW.

The EN line is called "Enable." This control line is used to tell the LCD that you are sending it data. To send data to the LCD, your program should make sure this line is low (0) and then set the other two control lines and/or put data on the data bus. When the other lines are completely ready, bring EN high (1) and wait for the minimum amount of time required by the LCD datasheet (this varies from LCD to LCD), and end by bringing it low (0) again.

The RS line is the "Register Select" line. When RS is low (0), the data is to be treated as a command or special instruction (such as clear screen, position cursor, etc.). When RS is high (1), the data being sent is text data which sould be displayed on the screen. For example, to display the letter "T" on the screen you would set RS high.

The RW line is the "Read/Write" control line. When RW is low (0), the information on the data bus is being written to the LCD. When RW is high (1), the program is effectively querying (or reading) the LCD. Only one instruction ("Get LCD status") is a read command. All others are write commands--so RW will almost always be low.

Finally, the data bus consists of 4 or 8 lines (depending on the mode of operation selected by the user). In the case of an 8-bit data bus, the lines are referred to as DB0, DB1, DB2, DB3, DB4, DB5, DB6, and DB7.

### AN EXAMPLE HARDWARE CONFIGURATION

As we've mentioned, the LCD requires either 8 or 11 I/O lines to communicate with. For the sake of this tutorial, we are going to use an 8-bit data bus--so we'll be using 11 of the 8051's I/O pins to interface with the LCD.

Let's draw a sample psuedo-schematic of how the LCD will be connected to the 8051.

As you can see, we've established a 1-to-1 relation between a pin on the 8051 and a line on the 44780 LCD. Thus as we write our assembly program to access the LCD, we are going to equate constants to the 8051 ports so that we can refer to the lines by their 44780 name as opposed to P0.1, P0.2, etc. Let's go ahead and write our initial equates:

DB0 EQU P1.0
DB1 EQU P1.1
DB2 EQU P1.2
DB3 EQU P1.3
DB4 EQU P1.4
DB5 EQU P1.5
DB6 EQU P1.6
DB7 EQU P1.7
EN EQU P3.7
RS EQU P3.6
RW EQU P3.5
DATA EQU P1

Having established the above equates, we may now refer to our I/O lines by their 44780 name. For example, to set the RW line high (1), we can execute the following insutrction:

SETB RW

LED:- LIGHT EMITTING DIODE

Opto-coupler :- it has one IR LED and a photo- transistor. One pin of the LED is connected to the MCU to get a signal (0 or 1) and the pin is given ground. When the signal from the MCU is 0, then LED emits light. This light will turn on the npn transistor. Collector of the transistor is grounded. Emitter is connected to the pnp transistor whose collector is connected to Vcc and emitter to the relay.

P in npn transistor means 1 and n means 0.

RELAY

NC (normally closed)

Phase                                                            NO (normally open)

vcc

Device

One point of the relay is given to phase and that point is connected to NC point. VCC is connected to the coil the relay. When vcc is given the coil gets magnetized and slowly phase is connected to NO point which is further connected to the device to be operated by the relay.Neutral must be given to the device.

USES OF P.W.M BASED SPEED CONTROLLER FOR DC MOTER ARE GIVEN AS :-

This circuit will use as –

- DC lamp dimmer

- Small motor controller

- It would make a great speed control for a solar powered electric train.

- It was widely used in industry pourpose.

- BIBLIOGRAPHY

- WWW.Google.com.

- www.8051projects.info/projects.asp