EYE WRITER

INDEX:

- Importance of eye writer

- Introduction to eye tracking

- History

- Tracker types

- Technologies and techniques

- Foveated imaging

- Eye tracking vs. gaze tracking

- Eye tracking in practice

- Eye tracking of younger and elderly people in walking

- Choosing an eye tracker

- Applications

- Commercial applications

Importance of EYE WRITER:

How do you communicate when your brain is active but your body isn't? The EyeWriter, a collaboration from the Ebeling Group, the Not Impossible Foundation and Graffiti Research Lab, uses low-cost eye-tracking glasses and open-source software to allow people suffering from any kind of neuromuscular syndrome to write and draw by tracking their eye movement and translating it to lines on a screen.

The device was created for Tony "Tempt" Quan, an L.A.-based graffiti artist who was diagnosed with Lou Gehrig's disease in 2003. After trying the Eye Writer — the first time he'd drawn anything since he was fully paralyzed — Quan said, "It feels like taking a breath after being held underwater for five minutes."

Eye tracking:

It is the process of measuring either the point of gaze (where we are looking) or the motion of an eye relative to the head. An eye tracker is a device for measuring eye positions and eye movement. Eye trackers are used in research on the visual system, in psychology, in cognitive linguistics and in product design. There are a number of methods for measuring eye movement. The most popular variant uses video images from which the eye position is extracted. Other methods use search coils or are based on the electrooculogram.

# History:

# In the 1800s, studies of eye movement were made using direct observations. In 1879 in Paris, Louis Émile Javal .This observation raised important questions about reading, which were explored during the 1900s: On which words do the eyes stop? For how long? When does it regress back to already seen words?

An example of fixations and saccades over text.

This is the typical pattern of eye movement during reading. The eyes never move smoothly over still text.

Edmund Huey built an early eye tracker, using a sort of contact lens with a hole for the pupil. The lens was connected to an aluminum pointer that moved in response to the movement of the eye. Huey studied and quantified regressions (only a small proportion of saccades are regressions), and show that some words in a sentence are not fixated.

The first non-intrusive eye trackers were built by Guy Thomas Buswell in Chicago, using beams of light that were reflected on the eye and then recording them on film. Buswell made systematic studies into reading and picture viewing.

In the 1950s, Alfred L. Yarbus did important eye tracking research and his 1967 book is very highly quoted. He showed the task given to a subject has a very large influence on the subject's eye movement. He also wrote about the relation between fixations and interest:

"All the records show conclusively that the character of the eye movement is either completely independent of or only very slightly dependent on the material of the picture and how it was made, provided that it is flat or nearly flat."

The cyclical pattern in the examination of pictures "is dependent not only on what is shown on the picture, but also on the problem facing the observer and the information that he hopes to gain from the picture."

This study by Yarbus (1967) is often referred to as evidence on how the task given to a person influences his or her eye movement.

"Records of eye movements show that the observer's attention is usually held only by certain elements of the picture.  Eye movement reflects the human thought processes; so the observer's thought may be followed to some extent from records of eye movement (the thought accompanying the examination of the particular object). It is easy to determine from these records which elements attract the observer's eye (and, consequently, his thought), in what order, and how often."

"The observer's attention is frequently drawn to elements which do not give important information but which, in his opinion, may do so. Often an observer will focus his attention on elements that are unusual in the particular circumstances, unfamiliar, incomprehensible, and so on."

"When changing its points of fixation, the observer's eye repeatedly returns to the same elements of the picture. Additional time spent on perception is not used to examine the secondary elements, but to reexamine the most important elements."

In the 1970s, eye tracking research expanded rapidly, particularly reading research. A good overview of the research in this period is given by Rayner.

In 1980, Just and Carpenter formulated the influential Strong eye-mind Hypothesis, the hypothesis that "there is no appreciable lag between what is fixated and what is processed". If this hypothesis is correct, then when a subject looks at a word or object, he or she also thinks about (process cognitively), and for exactly as long as the recorded fixation. The hypothesis is often taken for granted by beginning eye tracker researchers.

During the 1980s, the eye-mind hypothesis was often questioned in light of covert attention, the attention to something that one is not looking at, which people often do. If covert attention is common during eye tracking recordings, the resulting scan path and fixation patterns would often show not where our attention has been, but only where the eye has been looking, and so eye tracking would not indicate cognitive processing.

According to Hoffman, current consensus is that visual attention is always slightly (100 to 250 ms) ahead of the eye. But as soon as attention moves to a new position, the eyes will want to follow.

We still cannot infer specific cognitive processes directly from a fixation on a particular object in a scene.  For instance, a fixation on a face in a picture may indicate recognition, liking, dislike, puzzlement etc. Therefore eye tracking is often coupled with other methodologies, such as introspective verbal protocols.

## Tracker types:

## Eye trackers measure rotations of the eye in one of several ways, but principally they fall into three categories:

## One type uses an attachment to the eye, such as a special contact lens with an embedded mirror or magnetic field sensor, and the movement of the attachment is measured with the assumption that it does not slip significantly as the eye rotates. Measurements with tight fitting contact lenses have provided extremely sensitive recordings of eye the dynamics and underlying physiology of eye movement.

## The second broad category uses some non-contact, optical method for measuring eye motion. Light, typically infrared, is reflected from the eye and sensed by a video camera or some other specially designed optical sensor. The information is then analyzed to extract eye rotation from changes in reflections. Video based eye trackers typically use the corneal reflection (the first Purkinje image) and the center of the pupil as features to track over time. A more sensitive type of eye tracker, the dual-Purkinje eye tracker, uses reflections from the front of the cornea (first Purkinje image) and the back of the lens (fourth Purkinje image) as features to track. A still more sensitive method of tracking is to image features from inside the eye, such as the retinal blood vessels, and follow these features as the eye rotates. Optical methods, particularly those based on video recording, are widely used for gaze tracking and are favored for being non-invasive and inexpensive.

## The third category uses electric potentials measured with electrodes placed around the eyes. The eyes are the origin of a steady electric potential field, which can also be detected in total darkness and if the eyes are closed. It can be modeled to be generated by a dipole with its positive pole at the cornea and its negative pole at the retina. The electric signal that can be derived using two pairs of contact electrodes placed on the skin around one eye is called Electrooculogram (EOG). If the eyes move from the centre position towards the periphery, the retina approaches one electrode while the cornea approaches the opposing one. This change in the orientation of the dipole and consequently the electric potential field results in a change in the measured EOG signal. Inversely, by analyzing these changes in eye movement can be tracked. Due to the discrete information given by the common electrode setup two separate movement components – a horizontal and a vertical – can be identified. A third EOG component is the radial EOG channel, which is the average of the EOG channels referenced to some posterior scalp electrode. This radial EOG channel is sensitive to the saccadic spike potentials stemming from the extra-ocular muscles at the onset of saccades, and allows reliable detection of even miniature saccades.

## Due to potential drifts and variable relations between the EOG signal amplitudes and the saccade sizes make it challenging to use EOG for measuring slow eye movement and detecting gaze direction. EOG is, however, a very robust technique for measuring saccadic eye movement associated with gaze shifts and detecting blinks. Contrary to video-based eye-trackers, EOG allows recording of eye movements even with eyes closed, and can thus be used in sleep research. It is a very light-weight approach that, in contrast to current video-based eye trackers, only requires very low computational power, works under different lighting conditions and can be implemented as an embedded, self-contained wearable system. It is thus the method of choice for measuring eye movement in mobile daily-life situations and REM phases during sleep.

## Technologies and techniques:

The most widely used current designs are video-based eye trackers. A camera focuses on one or both eyes and records their movement as the viewer looks at some kind of stimulus. Most modern eye-trackers use contrast to locate the center of the pupil and use infrared and near-infrared non-collimated light to create a corneal reflection (CR). The vector between these two features can be used to compute gaze intersection with a surface after a simple calibration for an individual.

Two general types of eye tracking techniques are used:

- Bright Pupil

- Dark Pupil

1) bright pupil                                                                  2) dark pupil

Their difference is based on the location of the illumination source with respect to the optics. If the illumination is coaxial with the optical path, then the eye acts as a retro-reflector as the light reflects off the retina creating a bright pupil effect similar to red eye. If the illumination source is offset from the optical path, then the pupil appears dark because the retro-reflection from the retina is directed away from the camera.

Bright Pupil tracking creates greater iris/pupil contrast allowing for more robust eye tracking with all iris pigmentation and greatly reduces interference caused by eyelashes and other obscuring features. It also allows for tracking in lighting conditions ranging from total darkness to very bright. But bright pupil techniques are not effective for tracking outdoors as extraneous IR sources interfere with monitoring.

Eye tracking setups vary greatly; some are head-mounted, some require the head to be stable (for example, with a chin rest), and some function remotely and automatically track the head during motion. Most use a sampling rate of at least 30 Hz. Although 50/60 Hz is most common, today many video-based eye trackers run at 240, 350 or even 1000/1250 Hz, which is needed in order to capture the detail of the very rapid eye movement during reading, or during studies of neurology.

Eye movement is typically divided into fixations and saccades, when the eye gaze pauses in a certain position, and when it moves to another position, respectively. The resulting series of fixations and saccades is called a scan path. Most information from the eye is made available during a fixation, but not during a saccade. The central one or two degrees of the visual angle (the fovea) provide the bulk of visual information; the input from larger eccentricities (the periphery) is less informative. Hence, the locations of fixations along a scan path show what information loci on the stimulus were processed during an eye tracking session. On average, fixations last for around 200 ms during the reading of linguistic text, and 350 ms during the viewing of a scene. Preparing a saccade towards a new goal takes around 200 ms.

scan path

Scan paths are useful for analyzing cognitive intent, interest, and salience. Other biological factors (some as simple as gender) may affect the scan path as well. Eye tracking in HCI typically investigates the scan path for usability purposes, or as a method of input in gaze-contingent displays, also known as gaze-based interfaces.

# Foveated imaging:

16:1 compression. Foveated image with fixation point at Stephen F. Austin statue.

Foveated imaging is a digital image processing technique in which the image resolution, or amount of detail, varies across the image according to one or more "fixation points." A fixation point indicates the highest resolution region of the image and corresponds to the center of the eye's retina, the fovea. Foveated imaging is also commonly referred to as space variant imaging or gaze contingent imaging.

The location of a fixation point may be specified in many ways. For example, when viewing an image on a computer monitor, one may specify a fixation using a pointing device, like a computer mouse. Eye trackers which precisely measure the eye's position and movement are also commonly used to determine fixation points in perception experiments. When the display is manipulated with the use of an eye tracker, this is known as a gaze contingent display. Fixations may also be determined automatically using computer algorithms.

Some common applications of foveated imaging include imaging sensor hardware and image compression. For descriptions of these and other applications, see the list below.

### Foveated sensors

Foveated sensors are multi-resolution hardware devices that allow image data to be collected with higher resolution concentrated at a fixation point. An advantage to using foveated sensor hardware is that the image collection and encoding can occur much faster than in a system that post-processes a high resolution image in software.

### Simulation

Foveated imaging has been used to simulate visual fields with arbitrary spatial resolution. For example, one may present video containing a blurred region representing a scotoma. By utilizing an eye-tracker and holding the blurred region fixed relative to the viewer's gaze, the viewer will have a visual experience similar to that of a person with an actual scotoma. The figure on the right shows a frame from a simulation of a glaucoma patient with the eye fixated on the word "similar."

### Quality assessment

Foveated imaging may be useful in providing a subjective image quality measure. Traditional image quality measures, such as peak signal-to-noise ratio, are typically performed on fixed resolution images and do not take into account some aspects of the human visual system, like the change in spatial resolution across the retina. A foveated quality index may therefore more accurately determine image quality as perceived by humans.

### Image database retrieval

In databases that contain very high resolution images, such as a satellite image database, it may be desirable to interactively retrieve images in order to reduce retrieval time. Foveated imaging allows one to scan low resolution images and retrieve only high resolution portions as they are needed. This is sometimes called progressive transmission.

## Eye tracking vs. gaze tracking:

Eye trackers necessarily measure the rotation of the eye with respect to the measuring system. If the measuring system is head mounted, as with EOG, then eye-in-head angles are measured. If the measuring system is table mounted, as with scleral search coils or table mounted camera (“remote”) systems, then gaze angles are measured.

In many applications, the head position is fixed using a bite bar, a forehead support or something similar, so that eye position and gaze are the same. In other cases, the head is free to move, and head movement is measured with systems such as magnetic or video based head trackers.

For head-mounted trackers, head position and direction are added to eye-in-head direction to determine gaze direction. For table-mounted systems, such as search coils, head direction is subtracted from gaze direction to determine eye-in-head position.

## Eye tracking in practice:

A great deal of research has gone into studies of the mechanisms and dynamics of eye rotation, but the goal of eye tracking is most often to estimate gaze direction. Users may be interested in what features of an image draw the eye, for example. It is important to realize that the eye tracker does not provide absolute gaze direction, but rather can only measure changes in gaze direction. In order to know precisely what a subject is looking at, some calibration procedure is required in which the subject looks at a point or series of points, while the eye tracker records the value that corresponds to each gaze position. (Even those techniques that track features of the retina cannot provide exact gaze direction because there is no specific anatomical feature that marks the exact point where the visual axis meets the retina, if indeed there is such a single, stable point.) An accurate and reliable calibration is essential for obtaining valid and repeatable eye movement data, and this can be a significant challenge for non-verbal subjects or those who have unstable gaze.

Each method of eye tracking has advantages and disadvantages, and the choice of an eye tracking system depends on considerations of cost and application. There are offline methods and online procedures like Attention tracking. There is a trade-off between cost and sensitivity, with the most sensitive systems costing many tens of thousands of dollars and requiring considerable expertise to operate properly. Advances in computer and video technology have led to the development of relatively low cost systems that are useful for many applications and fairly easy to use. Interpretation of the results still requires some level of expertise, however, because a misaligned or poorly calibrated system can produce wildly erroneous data.

### Eye tracking while driving a car in a difficult situation:

Frames from narrow road eye tracking described in this section

The eye movement of  two groups of drivers have been filmed with a special head camera by a team of the Swiss Federal Institute of Technology: Novice and experienced drivers had their eye-movement recorded while approaching a bend of a narrow road. The series of images has been condensed from the original film frames to show 2 eye fixations per image for better comprehension.

Each of these stills corresponds approximately to 0.5 seconds in real time.

The series of images shows an example of eye fixations #9 to #14 of a typical novice and an experienced driver.

Comparison of the top images shows that the experienced driver checks the curve and even has Fixation No. 9 left to look aside while the novice driver needs to check the road

and estimate his distance to the parked car.

In the middle images the experienced driver is now fully concentrating on the location where an oncoming car could be seen. The novice driver concentrates his view on the parked car.

In the bottom image the novice is busy estimating the distance between the left wall and the parked car, while the experienced driver can use his peripheral vision for that and still concentrates his view on the dangerous point of the curve: If a car appears there he has to give way, i. e. stop to the right instead of passing the parked car.

### Eye tracking of younger and elderly people in walking:

Elderly subjects depend more on foveal vision than younger subjects during walking. Their walking speed is decreased by a limited visual field, probably caused by a deteriorated peripheral vision.

Younger subjects make use of both their central and peripheral vision while walking. Their peripheral vision allows faster control over the process of walking.

## Choosing an eye tracker:

One difficulty in evaluating an eye tracking system is that the eye is never still, and it can be difficult to distinguish the tiny, but rapid and somewhat chaotic movement associated with fixation from noise sources in the eye tracking mechanism itself. One useful evaluation technique is to record from the two eyes simultaneously and compare the vertical rotation records. The two eyes of a normal subject are very tightly coordinated and vertical gaze directions typically agree to within +/- 2 minutes of arc (RMS of vertical position difference) during steady fixation. A properly functioning and sensitive eye tracking system will show this level of agreement between the two eyes, and any differences much larger than this can usually be attributed to measurement error.

## Applications:

A wide variety of disciplines use eye tracking techniques, including cognitive science, psychology (notably psycholinguistics, the visual world paradigm), human-computer interaction (HCI), marketing research and medical research (neurological diagnosis). Specific applications include the tracking eye movement in language reading, music reading, human activity recognition, the perception of advertising, and the playing of sport. Uses include:

- Cognitive Studies

- Medical Research

- Laser refractive surgery

- Human Factors

- Computer Usability

- Translation Process Research

- Vehicle Simulators

- In-vehicle Research

- Training Simulators

- Virtual Reality

- Adult Research

- Infant Research

- Adolescent Research

- Geriatric Research

- Primate Research

- Sports Training

- FMRI / MEG / EEG

- Commercial eye tracking (web usability, advertising, marketing, automotive, etc)

- Finding good clues

- Communication systems for disabled

- Improved image and video communications

- Computer Science: Activity Recognition

### Commercial applications:

In recent years, the increased sophistication and accessibility of eye tracking technologies have generated a great deal of interest in the commercial sector. Applications include web usability, advertising, sponsorship, package design and automotive engineering. In general, commercial eye tracking studies function by presenting a target stimulus to a sample of consumers while an eye tracker is used to record the activity of the eye.

- Examples of target stimuli may include websites, television programs, sporting events, films, commercials, magazines, newspapers, packages, shelf Displays, consumer systems (ATMs, checkout systems, kiosks), and software. The resulting data can be statistically analyzed and graphically rendered to provide evidence of specific visual patterns. By examining fixations, saccades, pupil dilation, blinks and a variety of other behaviors researchers can determine a great deal about the effectiveness of a given medium or product. While some companies complete this type of research internally, there are many private companies that offer eye tracking services and analysis.

- The most prominent field of commercial eye tracking research is web usability. While traditional usability techniques are often quite powerful in providing information on clicking and scrolling patterns, eye tracking offers the ability to analyze user interaction between the clicks. This provides valuable insight into which features are the most eye-catching, which features cause confusion and which ones are ignored altogether. Specifically, eye tracking can be used to assess search efficiency, branding, online advertisements, navigation usability, overall design and many other site components. Analyses may target a prototype or competitor site in addition to the main client site.

- Eye tracking is commonly used in a variety of different advertising media. Commercials, print ads, online ads and sponsored programs are all conducive to analysis with current eye tracking technology. Analyses focus on visibility of a target product or logo in the context of a magazine, newspaper, website, or televised event. This allows researchers to assess in great detail how often a sample of consumers fixates on the target logo, product or ad. In this way, an advertiser can quantify the success of a given campaign in terms of actual visual attention.

- Eye tracking provides package designers with the opportunity to examine the visual behavior of a consumer while interacting with a target package. This may be used to analyze distinctiveness, attractiveness and the tendency of the package to be chosen for purchase. Eye tracking is often utilized while the target product is in the prototype stage. Prototypes are tested against each other and competitors to examine which specific elements are associated with high visibility and appeal.

the first poster that responds to people looking at it based on their eye movement

- One of the most promising applications of eye tracking research is in the field of automotive design. Research is currently underway to integrate eye tracking cameras into automobiles. The goal of this endeavor is to provide the vehicle with the capacity to assess in real-time the visual behavior of the driver. The National Highway Traffic Safety Administration (NHTSA) estimates that drowsiness is the primary causal factor in 100,000 police-reported accidents per year. Another NHTSA study suggests that 80% of collisions occur within three seconds of a distraction. By equipping automobiles with the ability to monitor drowsiness, inattention, and cognitive engagement driving safety could be dramatically enhanced. Lexus claims to have equipped its LS 460 with the first driver monitor system in 2006, providing a warning if the driver takes his or her eye off the road.

Since 2005, eye tracking is used in communication systems for disabled persons: allowing the user to speak, send e-mail, browse the Internet and perform other such activities, using only their eyes. Eye control works even when the user has involuntary movement as a result of Cerebral palsy or other disabilities, and for those who have glasses or other physical interference which would limit the effectiveness of older eye control systems.

Eye tracking has also seen minute use in auto focus still camera equipment, where users can focus on a subject simply by looking at it through the viewfinder.

CONCLUSION:

- It is a boon to the disabled people who are suffering from ALS(Amyotrophic Lateral Sclerosis ), and other degenerative neuromuscular diseases, using reliable, cost effective and creative technologies.

- It is used for various research & development and studies conducted in exploring many fields of science and technology.

- The position and the movement of the eye(or pupil) can be tracked and thus the desired result is visualized on the screen(monitor) provided, which would enable us to read human thoughts.

- A 100% efficiency has not been achieved yet but researchers sure assure that such a high efficiency eye writer is just around the corner.

- Soon these eye writers will be affordable for even the common man to his advantage.

REFERENCES:

- Huey, E.B. (1968). The psychology and pedagogy of reading. Cambridge, MA: MIT Press. (Originally published 1908)

- Buswell, G.T. (1937). How adults read. Chicago, IL: University of Chicago Press.

- Yarbus, A. L. Eye Movements and Vision. Plenum. New York. 1967 (Originally published in Russian 1962).

- Rayner, K. (1998) Eye movements in reading and information processing: 20 years of research. Psychological Bulletin, 124, 372–422.

- Carpenter, Roger H.S.; Movements of the Eyes (2nd ed.). Pion Ltd, London, 1988.

- Hoffman, J. E. (1998). Visual attention and eye movements. In H. Pashler (ed.), Attention (pp. 119–154). Hove, UK: Psychology Press.

- Bojko, A. & Stephenson, A. (2005). It's All in the Eye of the User: How eye tracking can help answer usability questions. User Experience, Vol. 4, No. 1.

- National Highway Traffic Safety Administration. Retrieved July 9, 2006.

- Pieters.R  & Wedel.M (2007). Goal Control of Visual Attention to Advertising: The Yarbus Implication, Journal of Consumer Research, 2007, 34 (August), 224–233.

- www.cogain.org/wiki/Eye_Trackers

- www.eyegaze.com