## CONTENTS

1. INTRODUCTION………………………………………

1.1 LITERATURE SURVEY

1.2 BLOCK DIAGRAM

2. SYSTEM REQUIREMENTS……………………………

3. HARDWARE IMPLEMENTATION……………………

3.1 BLOCK DIAGRAM

3.2 CIRCUIT DIAGRAM

4. SYSTEM SPECIFICATION……………………………

4.1 MICROCONTROLLER AT89C51

4.2 GPS COMMUNICATION

4.3 SERIAL COMMUNICATION WITH XBEE

4.4 RF WITH ENCODERS AND DECODERS

4.5 LCD 16*2

4.6 L293D

4.7 IR SENSOR/ PIR SENSOR

4.8 KIEL TOOL WITH EMBEDDED C

5. SYSTEM WORKING………….…………….…………

5.1 WORKING

5.2 FLOW CHART

APPLICATIONS………………………………….………

FUTURE ENHANCEMENTS……………………………

LIMITATIONS …………………………………….………

CONCLUSION……………………………………….……

BIBLIOGRAPHY………….………………………….…..

APPENDIX……………………………………………………….

# CHAPTER 1

INTRODUCTION

A robot is a machine designed to execute one or more tasks repeatedly, with speed and precision. An important aspect of robotic security systems is surveillance of specified area. Interesting application can be seen in robot scanning areas to find explosive devices. Asset and location protection systems using robots allow hands-free operation via pre-operational programming to response to external stimuli. Over the long haul, it is easy to see that security robots can provide significant cost savings, while they may never replace a human security professional. Others may need to approach an armed barricaded suspect or enemy combatant. Still others need to go into a nuclear reactor to check if all is well. Increasingly, security managers are turning to robots to help get the job done. These works mainly focus on target perception and identification and robot localization. It is very essential to have a robot during disaster conditions like Earthquake or Bomb blast, where we have to identify live human beings as quickly as possible to save life. In these situations, human rescuers must make quick decisions under stress, and try to get victims

to safety often at their own risk. They must gather the location information and status of victims and the stability of the structures as quickly as possible so that medics and firefighters can enter the disaster area and save victims. All of these tasks are performed mostly by human and trained dogs, often in very dangerous and risky situations. This is why since some years, mobile robots have been proposed to help them and to perform tasks that neither humans dogs nor existing tools can do.

To most people, embedded systems are not recognizable as computers. Instead, they are hidden inside everyday objects that surround us and help us in our lives. Embedded Technology predominates as it overcomes the drawbacks of all the existing mechanical and electronic systems. An embedded system is housed on a single microcontroller board with the programs stored in ROM. Embedded systems typically do not interface with the outside world through familiar personal computer interface devices such as a mouse, keyboard and graphic user interface. Instead, they interface with the outside world through unusual interfaces such as sensors, actuators and specialized communication links.

The novel idea of this project is to locate the human presence using the robot which is controlled through RF communication. If it detects any human presence or obstacle, it will trace the longitude and latitude position and will send to the destination LCD using XBEE communication. This project can find its application in military and security where without our presence we can get to know the status of the other side. This project can also be used to navigate around the disaster areas and try to find humans who need help. After finding the humans it transmits the location to the static section. The second part is the static section and the controller. Specially designed pyro electric sensor is used for human detection.IR proximity sensor is used for obstacle detection. The data obtained by the sensors in the dynamic section are displayed in the static section using Xbee and GPS technologies.

  - LITERATURE SURVEY:

The current state of mobile robotics is that most of the research has been focused on solving their issues indoors because of the slightly more predictable nature (e.g. flat horizontal floors, well-structured space partitioning, smaller scale, etc). Many of the interesting applications are outdoors where fewer assumptions can be taken for granted. They are usually too large to consider applying certain techniques that achieved success indoors.

In this project, we target outdoor urban environments. These environments pose their own unique set of challenges that differentiate them from both the indoor and the open-space outdoor landscapes. On the other hand, typical outdoor sensors, such as GPS, have problems with reception around buildings. The robot built by Carnegie Mellon researchers on the USAR team is capable of navigating the difficult terrain of a disaster site but lacks sensors for victim detection. The contribution of this work is to provide a sensor suite for human detection in the urban disaster environment and trace the victim’s location using GPS (Global Positioning System).

  - OUTLINE OF THE REPORT:

- Chapter 1 gives the introductory part of the project which deals with the main idea of what our project really is.

- Chapter 2 gives the block diagram and system working.

- Chapter 3 provides the system requirements for our project which gives details and overview of all the circuits and its details which we implemented in project.

- Chapter 4 deals with software development which gives the flowchart of system.

- Chapter 5 gives the details about the Applications, advantages and limitations of our project.

- Chapter 6 describes the conclusion and also gives the details for future scope.

CHAPTER 2

SYSTEM REQUIREMENTS

Following requirements have been identified after analysis:

2.1 SOFTWARE REQUIREMENTS:

2.1.1 KEIL µVISION3 IDE:

KeilµVision3 IDE (Integrated Development Environment) is a Windows based front end for assembler. Keil µVision3 is used for writing Microcontroller programs. Standard libraries are altered or enhanced to address the peculiarities of an embedded target processor. IDEs typically present a single program in which all development is done. This environment typically provides many features for authoring, modifying, compiling, deploying and debugging software. The aim is to abstract the configuration necessary to piece together command line utilities in a cohesive unit, which theoretically reduces the time to learn a language and increases developer productivity.

2.2 HARDWARE REQUIREMENTS:

2.2.1 MICROCONTROLLER (ATMEL 89C51):

This is an Atmel microcontroller, which we are using in our project. This chip is   100% compatible with Intel 8051 microcontroller applications.

2.2.2 LCD:

We are using Hitachi LCD with dot matrix type which supports 16x2= 32    characters

2.2.3 XBEE:

XBEE is a wireless module which works with 2.4GHz frequency, generally used for error free long transmission. It supports both unicast and broad caste mode.

2.2.4 RF communication:

This RF Communication works with the frequency of 433 MHz. It works under the Ultra high frequency mode. The modulation scheme used here is ASK (Amplitude modulation scheme) as the IC used in Receiver part RX3310A uses ASK technique to demodulate it. We are using HT12E in encoder side and HT12D in receiver side to support RF communication data.

2.2.5 GPS COMMUNICATION:

GPS Communication is used to extract the Longitude and latitude of the current place. We are using Holux GPS module to do so. GPS uses 24 satellites for this operation.

2.2.6 PIR SENSOR:

The PIR (Passive Infra-Red) Sensor is a pyroelectric device that detects motion by measuring changes in the infrared levels emitted by surrounding objects. This motion can be detected by checking for a high signal on a single I/O pin.

2.2.7 MOTOR DRIVER L293D:

L293D driver is used to control the motor directions. It can sustain 6v to 32v. So we can connect any type of DC motor which comes under this voltage range. Generally geared DC motors are used to interface with this type of driver.

2.2.8 IR SENSOR:

IR proximity sensor is designed using a 555 timer.IR sensor and a TSOP receiver. The IR transmitter and TSOP receiver is placed in such an order that, if any object passes by, the TSOP sensor will get the signal from IR transmitter.

CHAPTER 3

BLOCK DIAGRAM

3.1 Robot Section:

IR Obstacle detection

Human Motion sensor

Microcontroller

89c51

Power supply

Motor driver

GPS

RF Decoder

RF 433 Rx

Max232

Xbee

3.2 Receiver Section:

Microcontroller

LCD

MAX232

Power supply

XBEE

3.3 Controller Section:

RF TX

RF Encoder

CHAPTER 4

POWER SUPPLY

The power supply section is an important requirement for all units. It should deliver constant regulated supply for proper working of all units. A 12-0-12/500mA centre tap transformer is used for this purpose. The primary of the transformer is connected to mains through a switch. In the secondary side, the two end points are connected to two diodes to convert a 12V AC to a 12V pulsed DC. Here diodes form a full wave rectifier and capacitor of 1000μf / 25V is used for filtering purpose. The output of the capacitor is connected to input of IC7805 voltage regulator to get a constant output of 5V. The same output of capacitor is connected to the input of IC 7812 voltage regulator to get a constant output of 12V.

Full wave rectifier:

230v/24vv

Component specifications:

Transformer o/p - sine wave of 24v, since it is a 12-0-12 transformer.

Diode o/p - Full wave rectifier o/p with 24v.

Capacitor o/p - rippled DC.

7805 O/P - pure DC with 5v.

7812 O/P - pure DC with 12v.

Low power ICs of 78 series offers advantages such as good regulation, current limitation and short circuit protection at 100mA and thermal shut down in event of excess power dissipation. These regulators get damaged by incorrect polarity. The capacitor used at the output of regulator is to maintain the stability of the output.

CHAPTER 5

INTERFACING WITH THE 8051 MICROCONTROLLER

5.1 Microcontroller with LCD:

15                   16                                              7

LCD

1    2    3           6             5               4        14

P1.0

Microcontroller

P1.7

P2.0

P2.1

P2.2

VSS VCC VEE

Vcc                     Vss

LCD Pin Description:

| Pin | Symbol | I/O | Description |
| --- | --- | --- | --- |
| 1 | Vss | -- | Ground |
| 2 | Vcc | -- | +5V power supply |
| 3 | Vee | -- | Power supply to control LCD contrast |
| 4 | RS | I | RS=0 to select command code register,RS=1 to select data register |
| 5 | R/W | I | R/W=0 for write,R/w=1 for read |
| 6 | E | I/O | Enable |
| 7 | DB0 | I/O | The 8-bit data bus |
| 8 | DB1 | I/O | The 8-bit data bus |
| 9 | DB2 | I/O | The 8-bit data bus |
| 10 | DB3 | I/O | The 8-bit data bus |
| 11 | DB4 | I/O | The 8-bit data bus |
| 12 | DB5 | I/O | The 8-bit data bus |
| 13 | DB6 | I/O | The 8-bit data bus |
| 14 | DB7 | I/O | The 8-bit data bus |

Microcontroller interacts with LCD to display data on it. Port 0 pins are used to interact with the LCD. P 2.0  is connected to Register Select (RS) pin i.e. Pin 4 of LCD. If P 2.0 = 0, instruction commands code register is selected, allowing the user to send a command such as clear display, cursor at home etc. If P 2.0 = 1, the data register is selected, allowing the user to send data to be displayed on to the LCD.

P 2.2  is connected to Enable (EN) pin i.e. Pin 6 of LCD. The LCD to latch information presented on its data pins uses the enable pin. When data is supplied to data pins, a high-to-low pulse must be applied to this pin in order for the LCD to latch in the data present at the data pins. This pulse must be a minimum of 450 ns wide.

P 1.0: is connected to Data pin bit 0 of LCD.

P 1.1: is connected to Data pin bit 1 of LCD.

P 1.2: is connected to Data pin bit 2 of LCD.

P 1.3: is connected to Data pin bit 3 of LCD.

P 1.4: is connected to Data pin bit 4 of LCD.

P 1.5: is connected to Data pin bit 5 of LCD.

P 1.6: is connected to Data pin bit 6 of LCD.

P 1.7: is connected to Data pin bit 7 of LCD.

Vcc,Vss and Vee:

While Vcc and Vss provide +5V and ground respectively, Vee is used for controlling LCD contrast.

RS, register select:

There are two very important registers inside the LCD. The RS pin is used for their selection as follows. If RS = 0, the instruction command code register is selected, allowing the user to send a command such as clear display, cursor at home, etc. If RS = 1 the data register is selected, allowing the user to send data to be displayed on the LCD.

R/W, read/write:

R/W input allows the user to write information to the LCD or read information from it. R/W = 1 when reading; R/W = 0 when writing.

E,enable:

The enable pin is used by the LCD latch information presented to its data pins. When data is supplied to data pins, a high to low pulse must be applied to this pin in order for LCD to latch in the data present at the data pins.

Do-D7:

The 8-bit data pins, Do-D7, are used to send information to the LCD or read the contents of the LCD’s internal registers.

To display letters and numbers, we send ASCII codes for the letters A-Z, a-z, and numbers 0-9 to these pins while making RS=1.

The below Table lists the instruction command codes:

| Code (Hex) | Command to LCD Instruction Register |
| --- | --- |
| 1 | Clear display screen |
| 2 | Return home |
| 4 | Decrement cursor (shift cursor to left) |
| 6 | Increment cursor (shift cursor to right) |
| 5 | Shift display right |
| 7 | Shift display left |
| 8 | Display off, cursor off |
| A | Display off, cursor on |
| C | Display on, cursor off |
| E | Display on, cursor blinking |
| F | Display on, cursor blinking |
| 10 | Shift cursor position to left |
| 14 | Shift cursor position to right |
| 18 | Shift the entire display to the left |
| 1C | Shift the entire display to the right |
| 80 | Force the cursor to beginning of 1st line |
| C0 | Force the cursor to beginning of 2nd line |
| 38 | 2 lines and 5x7 matrix |

Table 3.2  LCD Command Codes

We use RS=0 to check the busy flag bit to see if the LCD is ready to receive information. The busy flag is D7 and can be read when R/W=1 and RS=0, as follows: if R/W=1, RS=0. When D7=1 (busy flag=1), the LCD is busy taking care of internal operations and will not accept any new information. When D7=0, the LCD is ready to receive new information.

Hence LCD forms the essential part of the project for displaying the user friendly messages for human interaction with the system at the receiver end.

5.2 Microcontroller with HT12D Receiver:

RF communication uses 433MHz to transmit the data with ASK communication. RF communication uses HT12D IC for decoding the data obtained at receiver end. The HT12D is a fully integrated, amplitude-shift-keying (ASK) modulation, single chip receiver. It is designed to operate in a variety of low power radio applications. All popular radio frequencies from 250 MHz to 450 MHz may be supported by simply choosing the appropriate external components. Particular emphasis has been placed on low current consumption.

18                 14

10

HT12D

1

2                           13

3                           12

4                           11

5                           10

6

7

8    9         15       16

RF 433 MHz

Vcc

Gnd

Din

Vcc

P1.7

P1.6

P1.5

P1.4

Microcontroller

Gnd

68k ohm

Features of HT12D

- Operating voltage: 2.4V~12V

- Low power and high noise immunity CMOS technology

- Low standby current

- Capable of decoding 12 bits of information

- Binary address setting

- Received codes are checked 3 times

- Address/Data number combination

- HT12D: 8 address bits and 4 data bits

- Built-in oscillator needs only 5% resistor

- valid transmission indicator

- Easy interface with an RF or an infrared transmission medium

- Minimal external components

- Pair with Holtek_s 212 series of encoders

- 18-pin DIP, 20-pin SOP package

The 2^12 decoders are a series of CMOS LSIs for remote control system applications. They are paired with Holtek_s 212 series of encoders (refer to the encoder/decoder cross reference table). For proper operation, a pair of encoder/decoder with the same number of addresses and data format should be chosen. The decoders receive serial addresses and data from a programmed 212 series of encoders that are transmitted by a carrier using an RF or an IR transmission medium. They compare the serial input data three times continuously with their local addresses. If no error or unmatched codes are found, the input data codes are decoded and then transferred to the output pins. The VT pin also goes high to indicate a valid transmission. The 212 series of decoders are capable of decoding information’s that consist of N bits of address and 12_N bits of data.

5.3 Microcontroller with Motor Driver:

The Device is a monolithic integrated high voltage, high current four channel driver designed to accept standard DTL or TTL logic levels and drive inductive loads (such as relays solenoids, DC and stepping motors) and switching power transistors. To simplify use as two bridges each pair of channels is equipped with an enable input. A separate supply input is provided for the logic, allowing operation at a lower voltage and internal clamp diodes are included. This device is suitable for use in switching applications at frequencies up to 5 kHz.

MICROCONTROLLER

P2.4

P2.3

P2.2

P2.1

P2.0

L293D

1                       3

9                       6

15

10

7                     11

2                     14

16

8

4  5  12  13

12V

M1

M2

Features of L293D:

- 1.2A PEAK OUTPUT CURRENT (non repetitive) PER CHANNEL

- ENABLE FACILITY

- 600mA OUTPUT CURRENT CAPABILITY PER CHANNEL.

- OVERTEMPERATURE PROTECTION

- LOGICAL "0" INPUT VOLTAGE UP TO 1.5 V

- HIGH NOISE IMMUNITY

- INTERNAL CLAMP DIODES

TRUTH TABLE:

The L293D is assembled in a 16 lead plastic package which has 4 center pins connected together and used for heat sinking The L293DD is assembled in a 20 lead surface mount which has 8 center pins connected together and used for heat sinking.

Each motor usually requires 3 pins (2 for direction, and 1 for enabling the motor). Connect the enable pin of L293D IC to +5V for saving the number of pins required and use only the direction pins to change directions and turn the motor ON or OFF. Setting the direction pin with high and low on the other direction will make the motor spin in one direction and vice versa. Just set both directions to low then you can turn your motor off.

5.4 Microcontroller with Max232:

3.2.3 RF Communication with HT12E Transmitter:

18                 17

10

HT12E

1

2                           13

3                           12

4                           11

5                           10

6

7

8    9         15       16

RF 433 Mhz

Vcc

Gnd

Dout

Vcc

Key1

Key2

Key3

Key4

Gnd

1.2M ohm

M ohm is connected to pin 15th and 16th. This acts as a RC oscillator. [Indicated in Data sheet]

3.2.4 RF COMMUNICATION WITH HT 12D RECEIVER:

3.2.5 MICROCONTROLLER AND MAX232:

Microcontroller

(TX)P3.1

DB-9 Connector

3   5

XBEE/GPS

CHAPTER 4

SYSTEM SPECIFICATION

4.1 Microcontroller AT89C51

- General description

- Hardware interactions with microcontroller

4.2   GPS

4.3   RF Communication with Encoders and decoders

4.4   LCD 16*2

4.5   SERIAL COMMUNCIATION WITH XBEE

4.6   IR

4.7   PIR

4.8   L293D

4.9   EMBEDDED C WITH KEIL

4.1 MICROCONTROLLER-ATMEL89C51

4.1.1General description:

This device is a Single-Chip 8-Bit Microcontroller manufactured in an advanced CMOS process and is a derivative of the 8051 microcontroller family. The instruction set is 100% compatible with the 8051 instruction set.

The Microcontroller AT89C51 is housed in a 40-pin DIP. The unique features of Microcontroller are:

- 4 Kbytes of On-chip Flash Program Memory

- In-Application Programming (IAP) capability

- Internal RAM of 128 bytes

- RAM expandable externally to 64 Kbytes

- two 16-bit Timer/Counters

- six interrupt sources

- Four interrupt priority levels

- Four 8-bit I/O ports

- Full-duplex enhanced UART

- Operating frequency:11.0592 MHz

- Power control modes

  - Clock can be stopped and resumed

  - Idle mode

  - Power down mode

Fig 4.1 Internal block diagram of AT89C51.

4.1.2 Pin Description:

Fig 4.2 Pin diagram of AT89C51

VCC: Supply voltage.

GND: Ground.

Port 0:

Port 0 is an 8-bit open drain bi-directional I/O port. As an output port, each pin can sink eight TTL inputs. When 1s are written to port 0 pins, the pins can be used as high-impedance inputs.

Port 0 can also be configured to be the multiplexed low-order address/data bus during accesses to external programmed data memory. In this mode, P0 has internal pull-ups. Port 0 also receives the code bytes during Flash programming and outputs the code bytes during program verification.

Port 1:

Port 1 is an 8-bit bi-directional I/O port with internal pull-ups. The Port 1 output buffers can sink/source four TTL inputs. When 1s are written to Port 1 pins, they are pulled high by the internal pull-ups and can be used as inputs. As inputs, Port 1 pins that are externally being pulled low wilt source current (IIL) because of the internal pull-ups. In addition, P1.0 and P1.1 can be configured to be the timer/counter 2 external count input (P1.0/T2) and the timer/counter 2 trigger input (P1.1/T2EX), respectively.

Port 2:

Port 2 is an 8-bit bi-directional I/O port with internal pull-ups. The Port 2 output buffers can sink/source four TTL inputs. When 1s are written to Port 2 pins, they are pulled high by the internal pull-ups and can be used as inputs. As inputs, Port 2 pins that are externally being pulled low will source current (IIL) because of the internal pull-ups. Port 2 emits the high-order address byte during fetches from external program memory and during accesses to external data memory that uses 16-bit addresses (MOVX @ DPTR). In this application, Port 2 uses strong internal pull-ups when emitting 1s. During accesses to external data memory that uses 8-bit addresses (MOVX @ RI), Port 2 emits the contents of the P2 Special Function Register.

##### Port 3:

Port 3 is an 8-bit bi-directional I/O port with internal pull-ups. The Port 3 output buffers can sink/source four TTL inputs. When 1s are written to Port 3 pins, they are pulled high by the internal pull-ups and can be used as inputs. As inputs, Port 3 pins that are externally being pulled low will source current (IIL) because of the pull-ups. Port 3 also serves the functions of various special features of the AT89C51.

Port Pin Alternate Functions:

P3.0 RXD (serial input port)

P3.1 TXD (serial output port)

P3.2 INT0 (external interrupt 0)

P3.3 INT1 (external interrupt 1)

P3.4 T0 (timer 0 external input)

P3.5 T1 (timer 1 external input)

P3.6 WR (external data memory write strobe)

P3.7 RD (external data memory read strobe)

Port 3 also receives some control signals for Flash programming and   programming verification. RST Reset input. A high on this pin for two machine cycles while the oscillator is running resets the device.

ALE/PROG:

Address Latch Enable is an output pulse for latching the low byte of the address during accesses to external memory. This pin is also the program pulse input (PROG) during Flash programming.

In normal operation, ALE is emitted at a constant rate of 1/6 the oscillator frequency and may be used for external timing or clocking purposes. Note, however, that

one ALE pulse is skipped during each access to external data memory. If desired, ALE operation can be disabled by setting bit 0 of SFR location 8EH. With the bit set, ALE is active only during a MOVX or MOVC instruction. Otherwise, the pin is weakly pulled high. Setting the ALE-disable bit has no effect if the Microcontroller is in external execution mode.

##### PSEN:

Program Store Enable is the read strobe to external program memory. When the AT89C52 is executing code from external program memory, PSEN is activated twice each machine cycle, except that two PSEN activations are skipped during each access to external data memory.

EA/VPP:

External Access Enable: A must be strapped to GND in order to enable the device to fetch code from external program memory locations starting at 0000H up to FFFFH. Note, however, that if lock bit 1 is programmed, EA will be internally latched on reset. EA should be strapped to VCC for internal program executions. This pin also receives the 12-volt programming enable voltage (VPP) during Flash programming when 12-volt programming is selected.

##### XTAL1:

Input to the inverting oscillator amplifier and input to the Internal clock operating circuit.

##### XTAL2:

Output from the inverting oscillator amplifier.

4.1.3 OSCILLATOR AND CLOCK:

The heart of the 89C52 is the circuitry that generates the clock pulses. Pins XTAL1 and XTAL2 are provided for connecting a resonant network to form an oscillator. Typically, a quartz crystal and capacitors are employed as shown in figure. The crystal frequency is the basic internal clock frequency of the micro controller.

To calculate the time ‘Tinst’ any particular instruction will take to be executed, find the number of cycles ‘C’,

T inst = (C*12d)/(crystal frequency)

An 11.0592 MHz crystal is connected to XTAL1 and XTAL2 to yield a cycle frequency of 921.6 KHz.

4.1.4 MICROCONTROLLER AND ULN2003AG INTERACTIONS:

The ULN2003AG consist of high voltage, high current Darlington arrays, and each containing seven open collector Darlington pairs with common emitters. Each channel rated at 500mA and can withstand peak currents of 600mA. Suppression diodes are included for inductive load driving and the inputs are pinned opposite the outputs to simplify board layout.

4.1.5 MICROCONTROLLER AND LCD INTERACTIONS:

4.1.6. MICROCONTROLLER AND MAX232 INTERACTIONS:

Microcontroller interacts with GSM module serially for sending and receiving SMS. Microcontroller has built in UART (Universal Asynchronous Receiver Transmitter), which will help in asynchronous serial communication with GSM modem. Serial communication takes place over RS232 cable.  RS232 and Microcontroller are not compatible as the voltage levels to represent a bit are different for both. Therefore MAX232 is used to convert the voltage levels of microcontroller to that of RS232 while sending data and the other way while receiving data.

4.1.7. SERIAL COMMUNICATION:

To communicate serially, the baud rate has to be set. Configuring the special function registers (SCON and TMOD) of the microcontroller can set this.

SCON: Serial Port Control Register (Bit –Addressable)

One cost effective way to communicate is to send and receive bits serially. Asynchronous serial communication is widely used for character-oriented transmission. In Asynchronous serial communication, the data is placed in between a start bit and one or two stop bits. This is called Framing. During serial data communication, register SUBF is used to hold data to be transmitted / received. SCON register controls data communication, PCON controls data rates.

SBUF is physically two registers. One is writing only and is used to hold data to be transmitted out of 89C52 via TXD. The other is read only and holds received data from external sources via RXD. A register called SCON at address 98H controls the serial port of microcontroller.

4.2 GPS

GR-87 GPS Receiver Module

Main Feature

􀁺 Build on high performance SiRF GSC3f/LP chipset.

􀁺 Average Cold Start time and under 42 seconds.

􀁺 Low power consumption

􀁺 20 channels “All-in-View” tracking.

􀁺 200,000+ effective correlators for fast TTFF and high sensitivity acquisitions

􀁺 Integrated ARM7TDMI CPU with software engineering services and available for embedded customer defined applications.

􀁺 on chip 1Mb SRAM.

􀁺 Dual TTL level serial ports with one for GPS receiver command message

Interface, another one for RTCM-104 DGPS input.

􀁺 Compact Board Size 1”x1”x0.27”(25.4x25.4x7mm) for easy integration into hand-held device.

􀁺 Reacquisition Time 0.1 seconds

􀁺 Support Standard NMEA-0183 and SiRF Binary protocol

􀁺 Support Accurate 1PPS output signal aligned with GPS Timing

􀁺 Multi-path Mitigation Hardware

􀁺 On-board WAAS /EGNOS Demodulator

􀁺 Built-in a lithium battery make GPS fast positioning

. Technical Specifications

2.1. Electrical Characteristics

2.1.1 General

1). Frequency：L1, 1575.42 MHz.

2). C/A code：1.023 MHz chip rate.

3). Channels：20 (according to SiRF’s spec.).

2.1.2 Accuracy (Open Sky)

1). Position：10 meters, 2D RMS.

2). 7 meters 2D RMS, WAAS corrected.

3). 1-5 meters, DGPS corrected.

4). Altitude ：< ± 35m Vertical in term of 95%.

5). Velocity：0.1 meters/second.

6). Time 1 microsecond synchronized to GPS time.

Datum

1). Default：WGS-84.

2). Other Support different datum by request.

2.1.4 Acquisition Rate (Open sky, stationary requirements)

1). Reacquisition：0.1 sec, average.

2). Hot start：1 sec, average.

3). Warm start：38 sec, average.

4). Cold start：42 sec, average.

Dynamic Conditions

1). Altitude: 18,000 meters (60,000 feet) max.

2). Velocity：515 meters/second (1000 knots) max.

3). Acceleration：4g, max.

4). Jerk：20 meters/second, max.

Power

1). Input power input：3.3 ~ 5.5 VDC input.

2). Input current: 50 mA (without antenna).

3). Backup power: 3V Rechargeable Lithium cell battery, up to 500 hours discharge.

RF interface

1). Antenna connector type: MMCX, output 3.0 VDC. (Optional output

VCC_IN)

2). Minimum signal tracking: -159 dBm (according to SiRF’s spec)

Serial Port

1). Two full duplex serial communication, CMOS 3.0V interface, with user selectable baud rate (4800-Default, 9600, 19200, 38400).

2). NMEA 0183 Version 2.2 & 3.0 ASCII output (GGA, GSA, GSV, RMC (VTG , GLL and ZDA for optional)).

3). SiRF binary-position, velocity, altitude, status output.

3.3.1 VCC_IN

This is the main DC power supply for a +3.3 ~ 5.5 V engine board.

3.3.2 TXA

This is the main transmitting channel and is used to output navigation and Measurement data for user written software.

3.3.3 RXA

This is the main receiving channel and is used to receive software commands to the board from user written software.

3.3.4 RXB

This is the auxiliary receiving channel and is used to input differential corrections to the board to enable DGPS navigation.

3.3.5 GND

GND provides the ground for the board. Connect all grounds.

4.3 RF COMMUNICATION WITH ENCODERS AND DECODERS.

RF communication uses 433MHz to transmit the data with ASK communication. RF communication uses RX3310A IC for decoding the data obtained at receiver end. Here are some of the features of RX3310A

The RX3310A is a fully integrated, amplitude-shift-keying (ASK) modulation, single chip receiver. It is designed to operate in a variety of low power radio applications. All popular radio frequencies from 250 MHz to 450 MHz may be supported by simply choosing the appropriate external components. Particular emphasis has been placed on low current consumption.

Features

􀂋 Wide frequency range: 250 MHz to 450 MHz

􀂋 High sensitivity

􀂋 Low power consumption

􀂋 Automotive temperature range

􀂋 High integration level requiring few and inexpensive external components

􀂋 SOP 18L package or SSOP 20L (150 mil) package

Applications

􀂋 Remote control systems

􀂋 Car alarm and other security systems

􀂋 Baby finder, wireless doorbell, wireless toys

Features of HT12E

_ Operating voltage

_ 2.4V~5V for the HT12A

_ 2.4V~12V for the HT12E

_ Low power and high noise immunity CMOS technology

_ Low standby current: 0.1_A (type.) at VDD=5V

_ Minimum transmission word

_ Four words for the HT12E

_ Built-in oscillator needs only 5% resistor

_ Data code has positive polarity

_ Minimal external components

_ HT12A/E: 18-pin DIP/20-pin SOP package

The 2^12 encoders are a series of CMOS LSIs for remote control system applications. They are capable of encoding information which consists of N address bits and 12_N data bits. Each address/ data input can be set to one of the two logic states. The programmed addresses/data are transmitted together with the header bits via an RF or an infrared transmission medium upon receipt of a trigger signal. The capability to select a TE trigger on the HT12E or a DATA trigger on the HT12A further enhances the application Flexibility of the 212 series of encoders.

Features of HT12D

_ Operating voltage: 2.4V~12V

_ Low power and high noise immunity CMOS technology

_ Low standby current

_ Capable of decoding 12 bits of information

_ Binary address setting

_ Received codes are checked 3 times

_ Address/Data number combination

_ HT12D: 8 address bits and 4 data bits

_ Built-in oscillator needs only 5% resistor

_ valid transmission indicator

_ Easy interface with an RF or an infrared transmission medium

_ Minimal external components

_ Pair with Holtek_s 212 series of encoders

_ 18-pin DIP, 20-pin SOP package

The 2^12 decoders are a series of CMOS LSIs for remote control system applications. They are paired with Holtek_s 212 series of encoders (refer to the encoder/decoder cross reference table). For proper operation, a pair of encoder/decoder with the same number of

addresses and data format should be chosen. The decoders receive serial addresses and data from a programmed 212 series of encoders that are transmitted by a carrier using an RF or an IR transmission medium. They compare the serial input data three times continuously with their local addresses. If no error or unmatched codes are found, the input data codes are decoded and then transferred to the output pins. The VT pin also goes high to indicate a valid transmission. The 212 series of decoders are capable of decoding information’s that consist of N bits of address and 12_N bits of data

4.4 LCD 16*2

Liquid crystal displays (LCD’s) offer several advantages over traditional cathode-ray tube displays that make them ideal for several applications. Of course, LCDs are flat and they use only a fraction of the power required by cathode-ray tubes. They are easier to read and more pleasant to work with for long periods of time than most ordinary video monitors. There are several tradeoffs as well, such as limited view angle, brightness, and contrast, not to mention high manufacturing cost.

16x2 LCD is used in this project to display data to user. There are two rows and 16 columns. It is possible to display 16 characters on each of the 2 rows. It has two registers, command register and data register.

4.5 RS 232 STANDARDS/XBEE

To allow compatibility among data communication equipment made by various manufacturers, an interfacing standard called RS232 was set by the Electronics Industries Association (EIA) in 1960. In 1963 it was modified and called RS232A. RS232B and RS232C were issued in 1965 and 1969, respectively.

However since the standards were set a long before the advent of TTL logic family its input and output voltage levels are not compatible TTL compatible. In RS232, -3 to –25 V represent a 1, while a 0 bit is +3 to +25 V making –3 to +3 undefined. For this reason, to connect any RS232 to a microcontroller system we must use voltage converter such as MAX232 IC chips to covert TTL logic levels to the RS232 voltage levels and vice versa.

RS232 (DB-9) Connector:

MAX 232:

Since the RS232 is not compatible with today’s microprocessor and microcontroller, we need a line driver to convert the RS232’s signals to TTL voltage levels that will be acceptable to the 89C52’s TxD and RxD pins. One example of such a converter is MAX232 from Maxim Corp. The MAX232 converts the RS232 voltage levels to TTL voltage levels, and vice versa. One advantage of the MAX232 chip is that it uses a +5V power source, which is the same as the source voltage for the 89C52, and Max 232, with no need for the dual power supplies that are common in many older systems.

Fig 5.3 Pin diagram of MAX232

The MAX232 has two sets of line drivers for transferring and receiving data. The line drivers used for TxD are called T1 and T2, while the line drivers for RxD are designated as R1 and R2. In many applications only one of each is used. For example, T1 and R1 are used together to TxD and RxD of 89C52, and the second set is left unused.

Description of pins used:

T1 IN:

This is an input pin. TxD of microcontroller is connected to this pin. The data to be sent to GSM modem is transmitted serially on this pin.

T1 OUT:

This is an output pin. This pin is connected to RxD of RS232. This pin transmits sends the data received from microcontroller serially to RS232.

R1 IN:

This is an input pin. This pin is connected to TxD of RS232. This pin receives data to be transmitted to microcontroller.

R1 OUT:

This is an output pin. This pin is connected to RxD of microcontroller. This pin sends the data received on R1 IN pin to microcontroller.

Vcc:

This pin is connected to +5v supply.

GND:

This pin is connected to ground.

XBEE

Features

High Performance, Low Cost

•Indoor/Urban: up to 100’ (30 m)

•Outdoor line-of-sight: up to 300’ (100 m)

•Transmit Power: 1 mW (0 dBm)

•Receiver Sensitivity: -92 dBm

Low Power

•TX Current: 45 mA (@3.3 V)

•RX Current: 50 mA (@3.3 V)

•Power-down Current: < 10uAmp

Worldwide Acceptance

FCC Approval (USA) Refer to Appendix A [p23] for FCC Requirements.

Systems that include XBee/XBee-PRO Modules inherit MaxStream’s Certifications.

ISM (Industrial, Scientific & Medical) 2.4 GHz frequency band

Manufactured under ISO 9001:2000 registered standards

XBee/XBee-PRO RF Modules are optimized for use in US, Canada, Australia, Israel

And Europe (contact MaxStream for complete list of approvals).

Mechanical drawings:

Specifications:

Serial communication:

The XBee/XBee-PRO OEM RF Modules interface to a host device through a logic-level asynchronous serial port. Through its serial port, the module can communicate with any logic and voltage compatible UART; or through a level translator to any serial device (For example: RS-232/485/ 422 or USB interface board).

Data enters the module UART through the DI pin (pin 3) as an asynchronous serial signal. The signal should idle high when no data is being transmitted.

Each data byte consists of a start bit (low), 8 data bits (least significant bit first) and a stop bit (high). The following figure illustrates the serial bit pattern of data passing through the module.

Pin configurations:

RS232 pins of XBEE

4.6 IR

IR proximity sensor is generally used to monitor any obstacles or objects. This type of sensor is built by a 555 timer, IR transmitter, TSOP receiver and passive components. The range of IR proximity will vary from 5cm to few meters. Here are the basic building blocks of the IR proximity sensor.

555 TIMER:

The LM555 is a highly stable device for generating accurate time delays or oscillation. Additional terminals are provided for triggering or resetting if desired. In the time delay mode of operation, the time is precisely controlled by one external resistor and capacitor. For astable operation as an oscillator, the free running frequency and duty cycle are accurately controlled with two external resistors and one capacitor. The circuit may be triggered and reset on falling waveforms, and the output circuit can source or sink up to 200mA or drive TTL circuits.

Features

- Direct replacement for SE555/NE555

- Timing from microseconds through hours

- Operates in both astable and monostable modes

- Adjustable duty cycle

- Output can source or sink 200 Ma

- Output and supply TTL compatible

- Temperature stability better than 0.005% per °C

- Normally on and normally off output

- Available in 8-pin MSOP package

Applications:

- Precision timing

- Pulse generation

- Sequential timing

- Time delay generation

- Pulse width modulation

- Pulse position modulation

- Linear ramp generator

ASTABLE OPERATION

If the circuit is connected as shown in Figure 4 (pins 2 and 6 connected) it will trigger itself and free run as a multivibrator. The external capacitor charges through RA + RB and discharges through RB. Thus the duty cycle may be precisely set by the ratio of these two resistors.

In this mode of operation, the capacitor charges and discharges between 1/3 VCC and 2/3 VCC. As in the triggered mode, the charge and discharge times, and therefore the frequency are independent of the supply voltage.

Applications Information

Figure shows the waveforms generated in this mode of operation.

VCC = 5V Top Trace: Output 5V/Div.

TIME = 20μs/DIV. Bottom Trace: Capacitor Voltage 1V/Div.

RA = 3.9kΩ

RB = 3kΩ

C = 0.01μF

The charge time (output high) is given by:

t1 = 0.693 (RA + RB) C

And the discharge time (output low) by:

t2 = 0.693 (RB) C

Thus the total period is:

T = t1 + t2 = 0.693 (RA +2RB) C

The frequency of oscillation is:

Below fig may be used for quick determination of these RC values.

The duty cycle is:

EXAMPLE OF 555:: PULSE WIDTH MODULATOR

When the timer is connected in the monostable mode and triggered with a continuous pulse train, the output pulse width can be modulated by a signal applied to pin 5. In below Figure are some waveform examples.

TSOP RECIEVER:

The TSOP17... – series are miniaturized receivers for infrared remote control systems. PIN diode and preamplifier are assembled on lead frame, the epoxy package is designed as IR filter. The demodulated output signal can directly be decoded by a microprocessor. TSOP17. Is the standard IR remote control receiver series, supporting all major transmission codes.

Features

_  Photo detector and preamplifier in one package

_  Internal filter for PCM frequency

_  Improved shielding against electrical field disturbance

_ TTL and CMOS compatibility

_ Output active low

_ Low power consumption

_ High immunity against ambient light

_ Continuous data transmission possible (up to 2400 bps)

_ Suitable burst length .10 cycles/burst

4.8 MOTOR DRIVER L293D

The L293 and L293D are quadruple high-current half-H drivers. The L293 is designed to provide bidirectional drive currents of up to 1 A at voltages from 4.5 V to 36 V. The L293D is designed to provide bidirectional drive currents of up to 600-mA at voltages from 4.5 V to 36 V. Both devices are designed to drive inductive loads such as relays, solenoids, dc and bipolar stepping motors, as well as other high-current/high-voltage loads in positive-supply applications.

All inputs are TTL compatible. Each output is a complete totem-pole drive circuit, with a Darlington transistor sink and a pseudo-Darlington source. Drivers are enabled in pairs, with drivers 1 and 2 enabled by 1,2EN and drivers 3 and 4 enabled by 3,4EN. When an enable input is high, the associated drivers are enabled and their outputs are active and in phase with their inputs. When the enable input is low, those drivers are disabled and their outputs are off and in the high-impedance state. With the proper data inputs, each pair of drivers forms a full-H (or bridge) reversible drive suitable for solenoid or motor applications.

On the L293, external high-speed output clamp diodes should be used for inductive transient suppression. A VCC1 terminal, separate from VCC2, is provided for the logic inputs to minimize device power dissipation. The L293and L293D are characterized for operation from 0°C to 70°C.

FEATURES:

􀀀 Wide Supply-Voltage Range: 4.5 V to 36 V

􀀀 Separate Input-Logic Supply

􀀀 Internal ESD Protection

􀀀 Thermal Shutdown

􀀀 High-Noise-Immunity Inputs

􀀀 Functional Replacements for SGS L293 and SGS L293D

􀀀 Output Current 1 A per Channel (600 mA for L293D)

􀀀 Peak Output Current 2 A per Channel (1.2 A for L293D)

􀀀 Output Clamp Diodes for Inductive Transient Suppression (L293D)

The Device is a monolithic integrated high voltage, high current four channel driver designed to accept standard DTL or TTL logic levels and drive inductive loads (such as relays solenoids, DC and stepping motors) and switching power transistors. To simplify use as two bridges each pair of channels is equipped with an enable input. A separate supply input is provided for the logic, allowing operation at a lower voltage and internal clamp diodes are included.

This device is suitable for use in switching applications at frequencies up to 5 kHz.

The L293D is assembled in a 16 lead plastic package which has 4 center pins connected together and used for heat sinking The L293DD is assembled in a 20 lead surface mount which has 8 center pins connected together and used for heat sinking.

4.6 PIR SENSOR

The PIR (Passive Infra-Red) Sensor is a pyroelectric device that detects motion by measuring changes in the infrared levels emitted by surrounding objects. This motion can be detected by checking for a high signal on a single I/O pin.

Features

 Single bit output

 Small size makes it easy to conceal

 Compatible with all Parallax microcontrollers

Features and Electrical Specification

Compact size (28 x 38 mm)

Supply current: DC5V-20V(can design DC3V-24V)

Current drain :< 50uA

(Other choice: DC0.8V-4.5V; Current drain: 1.5mA-0.1mA)

Voltage Output: High/Low level signal ：3.3V

(Other choice: Open-Collector Output)

TTL output

High sensitivity

Delay time：5s-18 minute

Blockade time：0.5s-50s (acquiescently 0 seconds)

Operation Temperature: -15oC -70Oc

Infrared sensor: dual element, low noise, high sensitivity

Light sensor: CdS photocell (can be add as customer requirement)

Theory of Operation

Pyroelectric devices, such as the PIR sensor, have elements made of a crystalline material that generates an electric charge when exposed to infrared radiation. The changes in the amount of infrared striking the element change the voltages generated, which are measured by an on-board amplifier. The device contains a special filter called a Fresnel lens, which focuses the infrared signals onto the element. As the ambient infrared signals change rapidly, the on-board amplifier trips the output to indicate motion.

Calibration

The PIR Sensor requires a ‘warm-up’ time in order to function properly. This is due to the settling time involved in ‘learning’ its environment. This could be anywhere from 10-60 seconds. During this time there should be as little motion as possible in the sensors field of view.

Sensitivity

The PIR Sensor has a range of approximately 20 feet. This can vary with environmental conditions. The sensor is designed to adjust to slowly changing conditions that would happen normally as the day progresses and the environmental conditions change, but responds by toggling its output when sudden changes occur, such as when there is motion.

4.7 ABOUT THE KEIL COMPILER

Keil Software (http://www.keil.com) publishes one of the most complete development tool suites for 8051 software, which is used throughout industry. For development of C code, their Developer's Kit product includes their C51 compiler, as well as an integrated 8051 simulator for debugging. A demonstration version of this product is available on their website, but it includes several limitations (see next section). This is the software that will be used for CECS-347.

The C programming language was designed for computers, though, and not embedded systems. It does not support direct access to registers, nor does it allow for the reading and setting of single bits, two very important requirements for 8051 software. In addition, most software developers are accustomed to writing programs that will by executed by an operating system, which provides system calls the program may use to access the hardware. However, much code for the 8051 is written for direct use on the processor, without an operating system. To support this, the Kiel compiler has added several extensions to the C language to replace what might have normally been implemented in a system call, such as the connecting of interrupt handlers.

The purpose of this manual is to further explain the limitations of the Keil compiler, the modifications it has made to the C language, and how to account for these in developing software for the 8051 micro controller

Sample code

- basic program to toggle bit Port0.5 10 times

Ans.      #include<reg51.h>

Sbit ssss=P0^5;

Main ()

{

Unsigned char a;

P0=0X00;// Make P0 as output port

For (a=0; a<10; a++)

{

ssss=0;   //toggling the bit

ssss=1;

}

Kiel tool

CHAPTER 5

SYSTEM WORKING

5.1 WORKING

5.2 FLOW CHART

5.1 WORKING (STEPS):

PROCEDURE:

- All the robotic controlling movements are done using RF wireless.

- From the transmitter end we are controlling the robotic action which will be sent to the receiver end via RF.

- If any obstacle is detected, the LCD at the transmitter end will receive intimation about it. This is done using Xbee transreciever pair which will be kept at both the end.

- Now in between the movements of the robot, if any human activities are observed by the human tracking sensor then it will trigger the GPS which will get the exact location of the place and will transmit this to the receiver end via Xbee.

Here we are using 8051 core microcontroller, which acts as a heart of the project. All the processing part is done using this microcontroller.. Display used here is of Liquid crystal display. MAX232 is used to convert the logic from RS232 to TTL Level or vise versa. GPS kit is used to track the current longitude and latitude from the satellite. Xbee of 2.4 GHz with Transreceiver logic is employed here RF encoders and decoders are used to track the signal obtained from RF 433. IR proximity is used to intimate the owner about the obstacles and human tracing sensors are used to indicate the presence of any human activity.

5.2 FLOW CHART

# APPLICATIONS

The concept of this robot can be expanded and used practically in the following industrial applications as well.

• Gas leakage Detector:

Can be implemented to detect the harmful leakage of the gas where no human can reach

• Security systems:

Can be implemented in industrial automation as a security robot

• Home automation:

All AC devices like fan, computer etc can be controlled using RF.

• Military applications:

This project can also be implemented in military application as a spy robo

FUTURE ENHANCEMENT

. We can also interface the VGA camera along with the Wireless Ethernet card, so that all the robotic movement can be controlled via internet.

.

# Limitations of the Project

Some of the limitations of this project are:

- This project is a microcontroller-based project. The main limitation is the size of ROM available on the chip for programming.

- Since we are using wireless technology for communication, RF Communication cannot be extended for some more distance.

- The module is reset if the power goes off.

CONCLUSION

BIBLIOGRAPHY

Literature references:

- “The 8051 Microcontroller and embedded systems”  by Muhammad Ali Mazidi and Janice Gillispie Mazidi., Sixth edition, Pearson Education

- “The 8051 Microcontroller architecture, programming and applications” by Kenneth J Ayala, second edition, Penram International Publishing.

- “Programming and Customizing The   8051 Micro controller” by Myke Predko.

Web sites:

- www.atmel/89c51/tutorials/architecture.com

- www.keil.com

- www.philips.com

- www.howstuffworks.com

- Introduction to Serial Communications, http://www.ctv.es/pckits/home.html

- Tutorials on 8051, www.8051.com

- www.datasheetcatalog.com

APPENDIX

ATMEL DATA SHEET

RELAY DRIVER ULN2003AG